COBRA-NET MODULE SPECIFICATION

Revision 1.1 Feb. 14, 2001

By: Dan Fraser

### All functions are to be controllable from an external PC.

#### Ability to select up to 8 of 32 signals present on the Ethernet input.

#### Ability to have up to 8 analog outputs.

#### Each output may be a full range audio signal or it may be processed in some way.

#### The 8 signals may be mixed in any combination to provide 1, 2 or more outputs or not mixed to give 8 full range outputs.

#### Any output can come from any of the 8 inputs or combination of inputs.

#### Typical configurations may include

stereo 3-way

stereo 2 or 3 way plus mono sub and/or center channel

mono 2 to 6 way

1 to 8 full range

We are trying to implement complex crossovers (multiple independent 2-pole filters), parametric EQ, signal alignment delay, advanced mid-filter protection limiters

Each output signal may have upper and lower frequency limits so the outputs may be used to drive multi-way speaker systems. Something like having 2 each LF, MF and HF main outputs, a sub-woofer output and a center channel output. Roll offs should be selectable between 12dB/oct and 24dB/oct. Also required for the entire signal is 12 band parametric EQ or individual 4 band parametric eq.

LF and sub channels require  time delay up to 800µs at 500Hz. Programmable time delay for the entire signal or other individual signals is also required.

We would like to control the unit's parametric EQ from a SMAART PRO® or similar window, while viewing dual FFT  measurements on screen.

#### There should be the capability to set gain and EQ for the output as a whole as well as for individual outputs.

#### There must be the ability to phase invert any of the output channels or all at once.

#### Each output must have the ability to incorporate signal limiting. Besides the threshold adjustment, the limiting ratio should be controllable. Desirable features would include adjustable attack and release times.

#### Binary Control Outputs are also required. These are two state lines, ON or OFF and may include: Remote power on, remote audio muting and wink function.

#### Common signals that have to be reported back to the PC include: high voltage and low voltage. and high temperature. These are analog signals reporting the state of the amplifier back to the administration PC. However, by common, we mean there will generally only be one set of these no matter how many outputs.

#### For each output channel, since these will go to different power amps, we need to read and report to the administration PC: input levels to each amplifier, signal output levels from each amplifier, current drawn by each amplifier and clip signals. 12 bit resolution is OK for these but 16 bit would be preferred.

#### QUESTIONS

#### Can the Cobranet use a single high quality D to A converter for up to 8 outputs with appropriate multiplexing or will up to 8 separate D/A converters be required?

#### To read analog signals from our amplifiers, can a single A/D with 8 or 16 analog inputs be used? Same for a standard MUX for reading binary feedback signals?

#### Can they give us a configuration for the hardware to deliver up to 8 24 bit analog outputs and 8 high/low outputs? Also to read and report back up to the amplifier up to 24 somewhat lower resolution [12 or 16 bit] non-audio analog signals and 16 high/low inputs. We would also need software routines to use with Visual Basic to send these signals and to process the data returned by them.

#### Are the settings held in flash RAM or other non-volatile memory?

#### Once the settings are made can they be software locked so they cannot be altered easily?

#### Can we define a subset of parameters that can be left unlocked while the rest are locked so we can allow users a limited range of control?

#### Can we have a secret backdoor password installed or some hardware method of bypassing the security? [eg. Shorting 2 pins together on a chip or something like that] We may need to make changes to settings at the factory when units are returned from the field and we do not know their passwords.

#### Can one or more of the analog inputs be sampled often enough, possibly at 16 bits resolution to provide talk back circuit(s) at reasonable quality?