SB_OCT FOR IMPATIENTS

REQUIREMENTS FOR USING SB_OCT:

Hardware:	PC with VGA display (640*480 pixel, 16 colors) and soundblaster compatible soundcard.

Operation System:	SB_OCT runs ONLY under DOS (> V5.0) and NOT in a DOS-window of WINDOWS. Therefore you have to close WINDOWS first (for WINDOWS 9X: Restart in DOS-mode).

Device driver:	If no DOS-driver for the soundcard is loaded (see entry in file CONFIG.SYS) you can use the WINDOWS-driver by first starting WINDOWS (this will load the WINDOWS-driver of the soundcard into memory) and then close it (trick: the driver is still in memory!).

INSTALLATION OF SB_OCT

REMARK:	Inputs in DOS-mode are emphasized by putting <brackets> around the expression. The brackets MUST be omitted during input!

- Copy SB_OCT.ZIP in any target directory on your harddisc (e.g. C:\SB_OCT)

- Unzip SB_OCT.ZIP (e.g. with the program UNZIP.EXE and the command <unzip sb_oct>)

- Change into target directory (e.g. C:\SB_OCT) and start SB_OCT with the command <sb_oct>

- Probably the BLASTER environment variable will not be set correctly. But SB_OCT will give some hints what to do. Usually the command <set blaster=a220 d0> that has to executed before starting SB_OCT will already work. If you are in doubt please check the configuration of the soundcard within WINDOWS or look into the documentation of your soundcard..

URGENT: PLEASE READ BEFORE 1st MEASUREMENT

Typical properties of soundcards:

- Max. sensitivity approx. 10 to 50 mV for microphone input and approx. 0.5 to 2 V for line input.

- Microphone inputs generally show a lower sensitivity for frequencies below 100 Hz in order to avoid overloads by words with explosive consonants.

- If voltages between GROUNDED equipment is to be measured it MUST BE OBSERVED IN ANY CASE that the signal mass of the used soundcard input (which is grounded by the PC!) must always be connected with the grounded side of the voltage to be measured (e.g. amplifier output). Otherwise the soundcard will be short circuited!

OPERATION OF SB_OCT

- Now SB_OCT should work. You can verify this by connecting a microphone to the mic input: if the level meter on the right hand side of the screen reflects the sound pressure level at the microphone position everything is OK. By the way: the mic input can be selected by pressing <I> (for input) and <M> (for mic), then the input sensitivity can be adjusted with the <+> or <-> key (or accepted by pressing <RETURN>), respectively (you should always try to reach at least +/- 25%  of the maximum input level on the level meter).

- In general SB_OCT asks the user for input in the lowest part of the screen (prompt area). A measurement can be started with <S> (for start) and will be stopped by once more pressing <S> (for stop).  The type and number of averages can be set by pressing <M> (for measurement; below the key to press will be set in brackets).

- It only left to explain the different possibilities the <F>ile menu offers.. First of all you can simply <S>ave a measurement). You can also load an already saved file for <C>omparison with the actual measurement. Finally a saved file can be used as <R>eference measurement. In this case only the deviation of the actual measurement to the saved reference measurement will be displayed. However, it is not possible to load a reference spectrum (-> display only deviations to reference spectrum) and load another saved file for comparison. This is only possible with the display and analysis program DISP_OCT (however, the demo version is only able to show the supplied demo files).

CALIBRATION OF SB_OCT

- A very important application of this feature is the frequency calibration of the soundcard. Because the frequency responses of the mic and line input may be different the frequency calibration must be performed for each input individually. Therefore a continuous noise signal (e.g. white or preferably pink noise from a test CD) must be measured e.g. directly at the CD player output (use maximum number of averages). This measurement must be saved (e.g. PNCD_L03.OCT for Pink Noise at the CD player output measured using the line input with 1/3rd octave band resolution). With pink noise excitation a soundcard with linear frequency response should show a perfect horizontal line whose level depends from many factors. If the just measured spectrum is stored and then loaded as reference only the deviation to the reference spectrum will be shown which must be 0 dB (logarithmic Y-axis) or 1 (linear Y-axis) for each frequency band in this special case. Possible frequency response errors of the soundcard, the CD player or of the excitation signal are now compensated.

- If the measurement is repeated with a different input sensitivity the new result will be parallel shifted, so the measurement result is depending from the used input (mic or line) and sensitivity setting (-X to 0 dB). This can be avoided if a level calibration of the soundcard (respectively the mixer) is performed. Therefore SB_OCT will be started with the option -c (so <sb_oct -c>). The user will be guided by comments at the lowest part of the screen  (prompt area) through the whole calibration process. During the calibration the output of the soundcard should provide a sinusoidal signal with 1000 Hz and approx. 60 mV (depending on the specific soundcard; the exact level of the signal must be measured e.g. with a digital multimeter). The output of the soundcard has to be connected at first with the line input and then SB_OCT automatically measures all input sensitivities of the soundcard. Subsequently the same procedure will be performed with the mic input. Now the output signal should be exactly 30 dB lower in level (this is 10^(-30/20)=0.0316 times the preceding value). Most soundcards realize this 30 dB decrease very exactly, but only very few voltage meters have a sufficient resolution in the 2mV-range, so preferably use the factor of 0.0316 instead of trusting a standard voltage meter! After successful completion SB_OCT creates a calibration file SB_OCTxx.CAL, where xx is the amplitude resolution of the soundcard (08 for 8-bit and 16 for 16-bit, some soundcards can be used in both resolutions depending on the BLASTER environment variable).

MEASURE IMPEDANCE WITH SB_OCT

- The last important feature of SB_OCT is the impedance measurement. This mode is initiated by starting SB_OCT with the command line option -i (so <sb_oct -i). Again the user will be guided by comments at the lowest part of the screen  (prompt area) through the whole process. Principally a resistor with similar value like the average expected value of the unknown impedance (e.g. 10 Ohm for a 8 Ohm loudspeaker with a minimum of 6.5 Ohm and a maximum of 25 Ohm) is connected in series with the unknown impedance. Successively the voltage across the known resistor and the unknown impedance is measured and then the unknown impedance is determined.
Please note: If voltages between GROUNDED equipment is to be measured it MUST BE OBSERVED IN ANY CASE that the signal mass of the used soundcard input (which is grounded by the PC!) must always be connected with the grounded side of the voltage to be measured (e.g. amplifier output). Otherwise the soundcard will be short circuited

FREQUENCY RESOLUTION OF SB_OCT

- Some basic settings SB_OCT (like e.g. the calibration or impedance mode) are activated by starting SB_OCT using special command line parameters. A list of all allowed parameters is displayed by typing <sb_oct -?>.  The settings for octave resolution (parameter -o) and blocksize (parameter -b) are of particular interest. The blocksize determines how many subsequently digitized time samples are used for the FFT calculation (Fast Fourier Transformation; calculates the frequency spectrum out of a time signal). With a blocksize of 4096 and the fixed sample frequency of 41667 Hz only 4096/41667 = 0.0983 seconds of the signal are used for the frequency calculation. This corresponds with a frequency resolution of 41667/4096 = 10.17 Hz. The whole frequency range up to 41667/2 = 20833 Hz is represented by 4096/2 bandpass filters with a bandwidth of 10.17 Hz each. This frequency resolution is not sufficient for the measurement of room resonances (usually several peaks in the frequency range between 30 to 100 Hz), even when a high octave resolution of e.g. 1/12 was chosen.  This can only be improved by increasing the blocksize to 8192 or even 16384.