harman/kardon         	 Service Bulletin

Service bulletin # H/K9805  Rev. 1  January 1999

To:	    All harman/kardon Service Centers

Models:  AVR65 AM/FM Receiver, North America (120v) version

Subject:  Hum or Buzz in loudspeakers

In the event you receive an AVR65 with the complaint of “There is a hum or buzz coming from the speakers in one or more channels”, perform the necessary steps listed below:

- Disconnect all external cables including power cord.

- Remove the (14) screws holding the top cover to the receiver; remove cover.

- Locate Fuses FU481 & FU482; they are on the Main PCB in the right front quadrant of the receiver.

- Remove and discard FU481 and FU482 and replace with h/k part# FS1067, (2A Slo-Blo 5 x 20mm GMC type).

- Replace the top cover; set the unit on its side on a padded surface.

- Remove the (8) screws holding the bottom grille screen in place.

- Locate power supply capacitors C481/C482 and IC482 (See diagrams); they are located in the right front quadrant of the Main PCB.

- Solder an insulated 16 - 20 gauge (0.5 to 1.0 mm2) jumper wire between the common (ground) point on C481/C482 (8200 63v) and the ground lead on IC482 (-15v regulator).

- Replace bottom screen; re-connect all external cables and test the receiver.

| Model | Serial number (120V) | Status | Action |
| --- | --- | --- | --- |
| AVR65 | JS0003-01001 to JS0003-07704 * | Needs Modification | Replace FU481 & FU482; Attach ground wire according to instructions |
| AVR65 | JS0003-07705 and above | Changed by factory | NONE REQUIRED |

* Any unit in this serial number range with a small green dot attached to the left of the serial number on the outer carton has been modified.