Last Update 1/17/2002; Event ID E15761658; Model DAVS300. Subject Satellite Speaker Assy.

SYMPTOMS

Satellite Speaker Assy available.

CAUSE

Speaker assemblies are now available. Since stock is limited the replacement speakers are to be used for exchange or replacement of missing speakers, not for repair. For repair of existing speaker use the part numbers in the service manual.

RESOLUTION

Use the part numbers outlined when ordering replacement speaker assemblies.

| Part Number | Reference Number | Description |
| --- | --- | --- |
| A4330691A | SS-TS300 | BOX ASSY, SPEAKER |

HCDS300. Subject DVD WILL NOT POWER UP - REPL OPEN L904.

SYMPTOMS

The screen went blank, audio worked. When you power up unit in DVD the waveform monitor shows the video is cycling on and off. In the test mode none of the servo tests worked but you have video and they system check runs without any errors.

RESOLUTION

Found open inductor, L904, on power supply board. This section supplies power to the motors. Replaced inductor, full checkout unit tested good.

| 141439811 | L904 | INDUCTOR |
| --- | --- | --- |

HCDS300. Subject NO AUDIO OUTPUT - REPLACED OUTPUT IC.

SYMPTOMS

No Sound

CAUSE

No audio output.

RESOLUTION

Replaced output IC TA2020-020. The unit functioned normally.

| 875963811 | IC331 | TA2020-020 |
| --- | --- | --- |

HCDS300. Subject WILL NOT DISPLAY SUB-TITL CAPTIONS ALL THE TIME OR LOCK UP.

SYMPTOMS

Sub-titles will freeze on screen intermittently.

RESOLUTION

Replaced DVD board resolved issue. Forwarding old board for evaluation.

| A4424426A | NA | DVD BOARD |
| --- | --- | --- |

HCDS300. Subject No audio and no video.

SYMPTOMS

No audio and no video

CAUSE

+10 and - 10 volts missing from CN903 on the power board. Found IC903 and IC904 were not being turned on. Switching transistor Q906 was not being turned on. There was 5v present on pin 8 of CN401 on the power board. Found open feed thru off of pin  4 of IC907.

RESOLUTION

Repaired feed thru unit tested good.

| A4428327A | NA | power board |
| --- | --- | --- |

HCDS300. Subject weak or no audio out.

SYMPTOMS

weak or no audio out

CAUSE

Unit had weak front right audio out. found L101 had bad solder connection.

RESOLUTION

Resoldered L101, unit back to normal operation.

| 141928111 | L101 | COIL |
| --- | --- | --- |

HCDS300. Subject dvd plays but no cd operation.

SYMPTOMS

dvd plays but no cd operation

CAUSE

The unit came in and would play a DVD fine but not play an audio cd. found IC001 defective.

RESOLUTION

Replaced IC001, unit restored to normal operation

| Part Number | Reference Number | Description |
| --- | --- | --- |
| 875959491 | IC001 | tk board |

HCDS300. Subject Replacement DVD board missing 0 ohm resistors.

SYMPTOMS

Replacement DVD board missing 0 ohm resistors

RESOLUTION

We continue to investigate this issue

| 1-676-634-21 |  | DVD Board |
| --- | --- | --- |

HCDS300. Subject owner's manual problem.

SYMPTOMS

owner's manual problem

CAUSE

Page 22 of owner's manual indicates HALL, JAZZCLUB, LIVE HOUSE, GAME, soundfields have center channel level adjustment. We have received numerous customer units with complaing no center channel in these modes.

RESOLUTION

This adjustment is impossible, there is no center channel in these modes.

HCDS300. Subject Part number missing for outer shell for front panel.

SYMPTOMS

Part number missing for outer shell for front panel

RESOLUTION

There are actually 2 parts that make up this part. The inner shell (X-4952-562-01) which is plastic and has the buttons on it, and the outer shell (4-226-328-01) which is metal. In the SM it appears as though this is one part, where actually it's two  separate parts.

| 4-226-328-01 | 52 | outer shell for front panel |
| --- | --- | --- |

HCDS300. Subject remote rms3300 does not allow direct channel entry for tv.

SYMPTOMS

according to the owners manual, per customer, the rems3300 remote should allow direct channel entry from the number pad, e.g. 1-1-enter, but this does not work. tried 2 replacement units same problem. is this a glitch in the o/m?

RESOLUTION

As per the Operator guide entering the correct codes for a given TV will allow the HCD-S300 remote to controll the TV. However, the disclaimer on page 52 states that Depending on the TV, you may not be able to control your TV or to use some of the  buttons above.

| 141883811 | NA | REMOTE |
| --- | --- | --- |

HCDS300. Subject Different power MCB.

SYMPTOMS

Different power MCB

CAUSE

1. Can we get same old part 2 is there any supplement manual for this, 3 new part has two extra cable but no place to connect and unit not working, need help asap

RESOLUTION

See Service bulletin #651R1 It shows how to use the old and new MCB with the TN board. In the diagram of section 2 it shows the 3 red wires connected to the same point for the boards with the extra conector.

| A4428327A | 8 | power mcb |
| --- | --- | --- |

HCDS300. Subject Part Correction in manual.

SYMPTOMS

The item ref #103 when ordered you recieve what appears to be the item shown as ref #115.

RESOLUTION

Ref #103 should be P/N 4-225-884-01 (Chassis) and ref #115 is 4-224-895-01 (Holder). Refer to Service manual p/n 9-929-072-14 for the correct part numbers.

| Part Number | Reference Number | Description |
| --- | --- | --- |
| 422588401 | 103 | chassis |
| 422489501 | 115 | Holder |

DAVS300, HCDS300. Subject Repeat failure of audio output IC.

SYMPTOMS

Repeat failure of audio output IC

CAUSE

Referring to SB651R1, the diagram indicates 6 IC301's?

RESOLUTION

This should be IC301/331/361 on both sides of diagram.

MORE INFORMATION

HCDS300. Subject Part number correction for Case Sub Assy.

SYMPTOMS

Reference #11 on page 71 of the service manual gives the top cabinet part number as X-4952-949-01 CASE SUB ASSY. This part number is incorrect.

RESOLUTION

Use part number X-4952-949-1