The three files "ptimrpwr.gif", "ptimrclk.gif", and "ptimrdsp.gif"

are schematics of a simple digital timer meant for photographic use.

There are three thumbwheel switches, with a seven-segment display

for each switch.  This is the timer setting.

There are power, range, start, and focus switches.  The power

switch is obvious.  Range chooses /1 or /10 operation.  For example,

if you dial "255", this means either 255 or 25.5 seconds, respectively.

Start begins the countdown, and turns on an AC outlet where an en-

larger would be optionally connected.  Focus turns the outlet on

continuously for as long as needed to set focus on the enlarger before

making an exposure.  The ranging feature lets the timer be used for

general purposes.

It is possible to adapt the circuit for 110 or 220 volt line power

easily.  The entire circuit is built from parts readily available; "no

exotic parts" was a specific design goal.  Construction is non-critical,

generally getting the thumbwheels mounted into a case properly is the

biggest hurdle.  To operate with a 50Hz line frequency, you must change

the pre-divider to divide-by-5 instead of six, or go to crystal control

using a specialty divider IC.

--------------------------\ /-----------------------------------------------

INTERNET:tindle@ms.uky.edu | "Could you please continue the petty bickering?

BITNET: tindle@ukma.bitnet |  I find it most intriguing."   ---    Data,

Ken Tindle - Lexington, KY |  Star Trek, The Next Generation, "Haven"

--------------------------/ \-----------------------------------------------