XFMR - THE PROGRAM

by Dan Fraser A.Sc.T.

FEATURES

1.   Will design single phase transformers of any power rating.

Has been successfully tested for transformers from 100VA to

5000VA.

2.   User sets input parameters for input voltage, frequency,

duty cycle, flux density, etc. However, it loads in standard

North American default settings unless user changes them.

The user can build a library of standard configurations of

input parameters.

3.   User may select a transformer lamination from the library of

standard sizes provided. User may add new sizes to the

library.

4.   The program uses long form calculation procedure with NO

shortcuts or rules of thumb.

5.   Fully menu driven.

6.   Outputs all relevant data including build information,

estimated weight, length of wire needed, estimated

efficiency and loads more.

7.   Program checks to make sure windings will fit into the

lamination's window and adjusts the design as necessary if

it will not fit initially.

8.   Tested in the design of over different 100 actually built

transformers.

TRANSFORMER DESIGN

STANDARD PROCEDURE FOR ALL TYPES

1.   Type XFMR (CR).

2.   On the prompt hit the space bar.

3.   Standard configuration is 110V 60 Hz input with 100% duty

cycle. If this is not suitable for you, use "SELECT NEW

CONFIGURATION FILE" and/or "ENTER NEW CONFIGURATION FILE" to

change the default settings. Only 110V 60 Hz with 100% duty

cycle configuration is available to non-registered users.

4.   If you wish to use a lamination other than "175", use

"SELECT CORE SIZE" to choose a new one. Only core size 175

is available to non-registered users.

a)   You are asked if you want to see the list. Enter "N" if

you know the number you want. Enter "Y" if you want to

see the list.

b)   Enter the part number indicated(this may differ from

the manufacturer's part number).

c)   Select the thickness desired (if in doubt, select "2"

as this is the most common.

d)   The computer will then draw the lamination (not to

scale). If this is correct, enter "Y".  If not, enter

"N" and the selection process will repeat.

5.   If the lamination you wish to use is not listed, use the

"ENTER NEW CORE SIZE" option to enter a new one. This is

fully menu driven. This is not available to non-registered

users.

6.   Select "DESIGN".

7.   You are then requested to indicate if the secondary winding

will be centre tapped. if it is, press "Y". Press "N" or

"ENTER" if not.

The configuration you have selected is displayed now.

VARIABLE PROCEDURES FOR DIFFERENT TYPES

STANDARD TRANSFORMERS WITH UNRECTIFIED OUTPUT

1.   Enter the desired full load output voltage.

2.   Enter the desired maximum output current.

3.   Wait until calculations are done and record the information

you need or press the "PRINT SCREEN" key for each of the two

information screens.

4.   Once done, the computer asks "Do another? (Y/N)"

BATTERY CHARGER TRANSFORMERS - FULL WAVE BRIDGE and SCR CONTROL

NOTE- this configuration is not recommended except for low

current chargers due to its very low efficiency. The Full Wave

Centre Tap with 2 SCR configuration has only 40% the heat

dissipation of this configuration.

1.   Select a configuration file with 75% duty cycle or when it

comes time to enter the current desired, multiply the figure

by 0.75. (DO NOT DO BOTH)

2.   After selecting your core and going to the DESIGN option,

enter a desired AC output voltage of approximately 150% of

the battery DC voltage (18 Volts AC for a 12V battery), or

the battery DC voltage plus 20 Volts, whichever option is

less.

3.   If the configuration selected is for 75% duty cycle, enter

the output current as the charger's nominal current rating.

If the duty cycle in the configuration selected is 100%,

enter a current rating of 75% of the charger's nominal

current rating.

BATTERY CHARGER TRANSFORMERS - FULL WAVE CENTRE TAP and 2 SCR

CONTROL

1.   Select a configuration file using 100% duty cycle.

2.   After selecting your core and going to the DESIGN option,

press "Y" when asked if a centre tapped transformer is

desired. Enter a desired AC output voltage of approximately

280% of the battery DC voltage (33 Volts AC for a 12V

battery), or the battery DC voltage plus 40 Volts, whichever

option is less.

3.   Enter the output current as 55% to 60% of the charger's

nominal current rating.

4.   When the build information is displayed, press "PRINT

SCREEN" to record the build information.

5.   Use two SCR's rated for the desired DC output current (each

SCR, on average, only sees 50% of the current).

POWER SUPPLY TRANSFORMERS - FULL WAVE BRIDGE and CAPACITOR FILTER

1.   Select a configuration file using 100% duty cycle.

2.   After selecting your core and going to the DESIGN option,

enter a desired AC output voltage of 71% of the desired DC

output voltage.

3.   Enter the output current as the desired DC output current

multiplied by 1.3.

4.   Use a bridge rectifier rated for at least 50% more current

than the desired DC output current and a filter capacitor of

at least 1000uFd. per amp of output.

POWER SUPPLY TRANSFORMERS - 2 DIODE FULL WAVE  and CAPACITOR

FILTER

1.   Select a configuration file using 100% duty cycle.

2.   After selecting your core and answering "Y" when asked if

the transformer will be centre tapped, enter a desired AC

output voltage of 142% the desired DC output voltage.

3.   Enter the output current as the desired DC output current

multiplied by .65.

4.   When the build information is displayed, press "PRINT

SCREEN" to record the build information.

5.   Use two diodes of two diodes of a bridge rectifier rated for

the desired DC output current (each diode, on average, only

sees 50% of the current) and a filter capacitor of at least

1000uFd. per amp of output.

VOLTAGE ADJUST TAPS

If you wish to have voltage adjustment taps on the primary,

it is suggested that a tap be provided at 8% under the full turns

for low line voltage and 8% additional winding be put on the

primary for a tap to compensate for excessive line voltage. This

is not required for most cases, especially not for SCR regulated

battery chargers.

220 VOLT TRANSFORMERS

After changing the configuration file for the correct

voltage and frequency, bifilar wind the primary with a centre

tap. This is so you can use a standard 110V neon pilot lamp and

so that you can temporarily test the unit on 110V power. The two

primary windings may then be series connected for 220V input and

paralleled for 110V operation.

For more information on transformer design and building,

read the book "PRACTICAL TRANSFORMER DESIGN HANDBOOK" published

by Howard W. Sams (Catalogue number 21657), available through

Active Components or other electronics book dealer.

DEFAULT VALUES

LAMINATION               # 175

BASE WIDTH               .875"

WINDOW LENGTH           2.6"

LEG WIDTH                .875"

I WIDTH                  .875"

WINDOW WIDTH             .875"

CENTRE WIDTH            1.75"

LAMINATION THICKNESS     .02"

INPUT VOLTAGE            110V

LINE FREQUENCY           60Hz

FLUX DENSITY             10000 GAUSS

CURRENT DENSITY          750 Circular mils/Amp

SPECIFIC WEIGHT          3

STACKING FACTOR          .9

BOBBIN THICKNESS         .1"

DUTY CYCLE               100%

BULGE FACTOR             .125%

MAIN MENU

1. DESIGN

2. SELECT NEW CONFIGURATION FILE

3. ENTER NEW CONFIGURATION FILE

4. SELECT CORE SIZE

5. ENTER NEW CORE SIZE

6. QUIT

OPTION 1 - DESIGN

IS SECONDARY 1 CENTRE TAPPED  (Y/N)

After selecting option 1 (DESIGN) and determining if you need a

centre tapped design, the screen will show the following display.

In our case, we will design a 110V to 22V transformer with

an 11 amp output.

CENTRE TAP               "YES or NO"

AC INPUT VOLTAGE         110 VOLTS AC

AC INPUT FREQUENCY       60 HERTZ

FLUX DENSITY             10000 GAUSS

CURRENT DENSITY          750 CIRCULAR MILS PER AMP

SPECIFIC WEIGHT          .3

DUTY CYCLE               100%

BULGE FACTOR             12.5%

LAMINATION SELECTED      175

You then enter

SECONDARY VOLTAGE        22

SECONDARY CURRENT        11

The computer then goes through the design process, adjusting the

transformer size to ensure that the windings will fit into the

laminations. If the windings are too thick, the program will

automatically increase the core thickness to allow for less turns

until the winding will fit.

The program will then generate two screens of data. The first

page is titled "BUILD SHEET FOR A  22Volt 11Amp TRANSFORMER"

In the upper left corner is a build diagram.

Non centre tapped               Centre tapped

PRIMARY    SECONDARY            PRIMARY   SECONDARY

���������Ŀ�����������                    �����������

110 V    ��  22 V                        �  11 V

60 Hz    ��  11 A              ��������Ŀ�  26 Turns

251  T   ��  53 Turns            110 V  ��  12 Ga

18 Ga    ��  12 Ga               60 Hz  ������������

����������������������            251 T  ������������

18 Ga  ��  11 V

���������ٳ  26 Turns

�  12 Ga

�����������

The large brackets are standard transformer industry

notation for a winding and the information in between is the

required build information.

To the right of the build diagram is the  "PHYSICAL BUILD"

listing. The first column is the estimated thickness for the

primary while the second column is the estimated thickness for

the secondary.

COPPER         .217      .272

PAPER          .0168     .012

COVER          .021      .019

BULGE          .032      .037

--------  --------

TOTAL          .289      .341

SPACE LEFT IN WINDOW =

.144     inches

"COPPER" is the estimated thickness of the primary and

secondary windings.

"PAPER" is the estimated thickness of the inter winding

paper layers.

"COVER" is the estimated thickness of the cover over the

windings.

"BULGE" is the estimated amount the windings will be larger

than theoretically possible. The more skilled you are at winding,

the lower you will be able to make this.

"TOTAL" is the estimated total winding thickness you will

achieve.

"SPACE LEFT IN WINDOW" is the estimated room left in the

lamination's window after the winding is inserted into it. This

is the amount of room you to play with if you are a sloppy

winder.

Below the build diagram, you have displayed

DUTY CYCLE               100%      This is the design duty cycle.

LAMINATION OR CORE NUMBER = 175    As selected by you

LAMINATION THICKNESS =   .02  INCHES    As selected by you

STACK THICKNESS =        1.61 INCHES    Estimated thickness of

laminations.

LAMINATIONS REQUIRED =   72

IRON WEIGHT =            7.99 lbs.      Laminations

COPPER WEIGHT =          1    lbs.      Wire

OTHER WEIGHT =           .9   lbs.      Paper, cover and bobbin

----------

TOTAL WEIGHT =           9.9  lbs.      Estimated total

You then press the space bar to get the second screen of data.

PRIMARY   SECONDARY

-------   ---------

VOLTAGE                  110 V     22 V

CURRENT                  2.44 A    11 A

TURNS                    251       53

GAUGE                    18        12 AWG

TURNS/IN.                22        10

MARGINS                  .18       .2 INCHES

WINDING LENGTH           2.24      2.19 INCHES

TURNS/LAYER              49        22

AVERAGE TURN LENGTH =    6.69      10.42 INCHES

No. OF LAYERS            6         3

WIRE LENGTH              140       46 FEET

ACTUAL FLUX DENSITY =    10000 GAUSS

BOBBIN THICKNESS =       .1 INCHES

ESTIMATED EFFICIENCY =   90 %

POWER DISSIPATION =      28 WATTS

ESTIMATED REGULATION =   5.13 %

"GAUGE" is the AWG size of the magnet wire required.

Available in all gauge numbers in small quantities at most any

motor rewinding shop.

"TURNS/IN." is the expected number iof turns you should get

per inch of bobbin if you wind neatly.

"MARGINS" is the space to be left between the end of the

winding and the bobbin. Never wind the wire directly to the end

of the available space to allow for thermal expansion.

"WINDING LENGTH" is the expected length of a neat winding

layer. Do not exceed this length if you expect the winding to fit

into your lamination.

"TURNS/LAYER" is the turns that you should get into your

winding length if you are neat.

"AVERAGE TURN LENGTH" is the amount of wire used for each

turn for that layer. The secondary is longer as the primary

winding is always the inner winding and the sceondary is the

outer.

"WIRE LENGTH" is the estimated length of wire needed for

each winding. However, do not cut off this exact length. Wind

each coil directly from your roll of wire. Wires cut precisely to

a predicted length are always a tiny bit too short.

"ACTUAL FLUX DENSITY" is the calculated flux density the

transformer will have. Compare this to the flux density in your

initial configuration. The two figures should not disagree too

much.

"BOBBIN THICKNESS" is the thickness the calculations have

allowed for to determine if the winding will fit. This is a

typical thickness for a plastic bobbin. A fibreboard bobbin will

probably be thinner. If you are using a thicker bobbin, adjust

the configuration files.

"POWER DISSIPATION" is the estimated loss in the transformer

at full current. This includes resistive heating as well as core

magnetization current.

"ESTIMATED REGULATION" is the amount the secondary voltage

should rise above the full load voltage at zero load.

OPTION 2 - SELECT NEW CONFIGURATION FILE

After selecting this option, the program will print

"CONFIGURATION NUMBER" and display the current configuration in

use. The data presented includes: VOLTAGE IN, LINE FREQUENCY,

DUTY CYCLE, CURRENT DENSITY, FLUX DENSITY, SPECIFIC WEIGHT and

BULGE FACTOR.

The program then asks "IS THIS THE CONFIGURATION YOU WANT?

(Y/N?)". Press "N" if you want a different one. You will then be

asked to input a new CONFIGURATION NUMBER.

The configuration will be displayed with the same data

fields as above. Once you have fould the correct one, press "Y"

when asked "IS THIS THE CONFIGURSATION YOU WANT?".

If no configursations suit you, select any configuration to

return to the main menu then select option 3 to create a new

configuration.

OPTION 3 - ENTER NEW CONFIGURSTION FILE

You are asked the following information

CONFIGURATION NUMBER               Enter a number from 1 to 40.

If the number is in use already, the program will ask "OVERWRITE

(Y/N?)". Answer "Y" if you want to change an existing file.

The program will then display your last used AC INPUT VOLTAGE. If

satisfactory, in the new file press ENTER. If you want a new AC

INPUT VOLTAGE, enter it and press ENTER. The default value it

110V

Similarly, enter AC LINE FREQUENCY, DUTY CYCLE, CURRENT DENSITY,

FLUX DENSITY, SPECIFIC WEIGHT and BULGE FACTOR.

If in doubt about any of these, press ENTER and use the default

value.

"AC LINE FREQUENCY" = 60 Hertz in North America and 50 Hertz

almost everywhere else.

"DUTY CYCLE" should be 100% in most applications. Exceptions

include battery chargers and power supplies for transceivers.

"CURRENT DENSITY" is defined in circular mils per amp. This

means the amount of wire cross sectional area for each amp of

current flow. 750 is a good conservative figure. 500 is usable in

a few light duty applications. The smaller this figure, the

smaller, lighter and cheaper the finished transformer but on the

other hand, it will run hotter and be more likely to fail. If you

are not a professional electrical engineer, use at least 750. If

you are a bit paranoid or just a reliability freak, use 1000.

"FLUX DENSITY" is the number of lines of magnetic flux in

the core. Unless you are a real pro, leave this at 10,000.

"SPECIFIC WEIGHT" is the weight in pounds per cubic inch of

the lamination. ".3" is a good typical number to work with.

"BULGE FACTOR" is the amount by which the windings will be

thicker than their theoretical amount. Try the default value of

12.5%. If you find that your windings are too tight, increase

this factor in future designs. If you are a real neat freak, you

might be able to get this to 8%, but don't try it starting out.

You are then asked if you want to do another configuration.

OPTION 4 - SELECT CORE SIZE

After selecting this option, you are asked "DO YOU WANT A LISTING

OF THE CORE SIZES IN STOCK? (Y/N?)". Press "Y" if you do.

Then, select the part number of the core you want.

The program outputs the following information to you:

LAMINATION NUMBER        BASE WIDTH          WINDOW LENGTH

I WIDTH                  LEG WIDTH           WINDOW WIDTH

CENTRE WIDTH             THICKNESS 1         THICKNESS 2

THICKNESS 3              THICKNESS 4         THICKNESS 5

Note that thicknesses 3,4 & 5 are only displayed if these

thicknesses are entered in the file for that core. Not all cores

are available in all thicknesses.

Then, you have to "SELECT THICKNESS DESIRED". Press the number

for the desired lamination thickness.

The program then prints out a dimensioned diagram of your

lamination (not to scale).

�#             CORE LENGTH               �

�����Ŀ   ������������������������������Ŀ

�     �   �                              �

�     �   �������������������������Ŀ    �

�#I   �                             �    �

�WIDTH�   �#   WINDOW LENGTH        �    �

�     �   ���������������������������    �

�     �   �    CENTRE WIDTH #            �

�     �   �������������������������Ŀ    �

�     �        WINDOW WIDTH   #     �    �

�     �                       #     �#   �BASE WIDTH

�     �   ���������������������������    �

�     �   �    LEG WIDTH   #             �

�������   ��������������������������������

It will also print out "LAMINATION THICKNESS = " and "LAMINATION

NUMBER = " and your selections.

This is followed by "IS THIS THE LAMINATION YOU WANT? (Y/N?)".

Press "Y" if it is correct.

OPTION 5 - INPUTING A NEW CORE SIZE

You are asked the following information

PART           Enter a number from 1 to 99. If the number is in

use already, the program will say "EXISTING LAMINATION NUMBER".

It then asks "OVERWRITE (Y/N?)". Answer "Y" if you want to change

an existing number.

LAMINATION NUMBER   Enter the manufacturer's number

EXISTING BASE WIDTH      This is the last used base width

BASE WIDTH               Enter a new base width or press ENTER if

the  existing base width is correct. The program will then

confirm the base width you entered.

Follow the same procedure  for WINDOW LENGTH, I WIDTH,

WINDOW WIDTH, LEG WIDTH, CENTRE WIDTH, THICKNESS 1, THICKNESS 2,

THICKNESS 3, THICKNESS 4 and THICKNESS 5.