Table of Contents

Table of Contents	Ⅱ

1.SYSTEM OVERVIEW	1-1

1.1 Introduction	1-1

1.2. Operational Specification	1-1

1.2.1 Environment	1-1

1.2.2 Vibration	1-2

1.2.3 Drop test	1-2

1.2.4 Packaging Configuration	1-3

1.2.5 Electrostatic Discharge Requirements	1-6

1.2.6 Safety Requirements	1-6

1.2.7 EMI Requirements	1-7

1.2.8 Acoustical Noise	1-7

1.2.9 Reliability	1-7

1.2.10 Mechanical Design for TCO 99	1-7

1.2.11 Environmental Protection Design	1-8

1.3. Input / Output Signal Specification	1-8

1.3.1 Input Signal Requirements	1-8

1.3.2 Function	1-9

1.3.3 Number of display colors	1-10

1.3.4 Adjustment function 	1-10

1.3.5 Plug & Play DDC1/2B Support 	1-10

1.3.6 Power Supply Requirements	1-12

1.3.7 LCD panel optical Characteristics	1-14

1.4. Functional Specification

1.4.1 Display Quality	1-15

1.4.2 Audio Quality	1-16

1.5.Physical Specifications	1-16

1.5.1 Physical Dimension & Appearance	1-16

1.5.2 Construction and Materials on outer surface	1-16

1.5.3 Base 	1-16

1.5.4 Marking & Labels	1-16

1.5.5 Packaging	1-17

1.6. Maintainability Specification	1-17

1.6.1 General Requirements	1-17

1.6.2 Mean Time to Repair	1-17

1.6.3 Accessibility	1-17

1.6.4 Equipment & Tools Required	1-18

2.CIRCUIT OPERATION THORY	2-1

2.1 Introduction	2-1

2.2 Block diagram 	2-1

2.3 Circuit operation theory	2-1

3.ALIGNMENT PROCEDURE	3-1

4.TROUBLE SHOOTINGV	4-1

4.1 No Picture or Picture is unstable	4-1

APPENDIX A	A-1

Chapter 1

SYSTEM OVERVIEW

1.1 Introduction

This specification describes an 18.1" analog interface color TFT LCD monitor.  It can support maximum      resolution 1280X1024 and the DPMS function .

It has the following features:

- DPMS (Display Power Management System)

- Flywheel control with push button function (for OSD & function control)

- Intelligent key  (Automatic Adjustment Function)

- Multi-scan support

- Power on/off indicator

- Universal AC power input

- 0.2805(W)*0.2805(H) mm pixel pitch LCD panel

- Speaker (1W/ch) with Volume control

- Microphone (S/N ratio =40db, Voltage=4.5 V)

- USB hub (optional)

It is composed of the following materials:

-A LCD monitor with a trailing 15 pin D-sub signal cable

(1) On the upper main bracket,

Include a piece of 18.1” TFT LCD panel, inverter board, interface board, DC-DC board,

control board, and microphone.

(2) One the lower base bracket,

Include a piece of AC-DC board, audio board, two speakers and a optional USB hub.

-A power cord

-A speaker cable

-A microphone cable

-Tilt base

-A USB cable(optional)

1.2 Operational Specification

1.2.1 Environment

1.2.1.1 Temperature

-Operating	5 to +35 degrees Celsius 
-Non-operating	-20 to +60 degrees Celsius

NOTE: If tested without packing, the max Non-operating temperature is 52 degree Celsius

1.2.1.2 Humidity

-Operating	20% to 85% non-condensing
-Non-operating	10% to 85% non-condensing

1.2.1.3 Altitude

-Operating	0  to  3,048m (10,000ft)

-Non-operating	0  to 12,192m (40,000ft)

1.2.2 Vibration

1.2.2.1 Sinusoidal Vibration

10 - 200 - 10Hz, 1.5G, 0.29 Oct/min, 30 min, X, Y, Z Axis.Each One time.

After test, there is no electrical and mechanical damage permitted.

1.2.2.2 Random Vibration

| Frequency(Hz) | Slope(dB/Oct.) | Spectrum Level(g2/Hz) |
| --- | --- | --- |
| 10~350 | 0 | 0.002 |
| 350~500 | -6 | ---------------- |

Equivalent to 1.0 Grms

*Duration: 30 Minutes/ Per Axis (X, Y, and Z Axis)

After test, there is no electrical and mechanical damage permitted.

1.2.3 Drop Test (with packing)

1.2.3.1 Drop Height

| Weight ( Kg ) | API Spec. Height (cm) |
| --- | --- |
| 0 - 9 | 91 |
| > 9 - 18.2 | 76 |
| > 18.2 - 27.2 | 61 |
| > 27.2 - 45.4 | 46 |
| > 45.4 - 68.1 | 31 |
| > 68.1 - 113.5 | 26 |

1.2.3.2 Drop Sequence

-Corner 5-3-2 select at weakness side [the low left(or right) corner of the front panel]

-An edge drop with impact on the shortest edge radiating from corner 5-3-2

-An edge drop with impact on the next shortest edge radiating from corner 5-3-2

-An edge drop with impact on the longest edge radiating from corner 5-3-2

-A flat drop with impact on the rear

-A flat drop with impact on the front

-A flat drop with impact on the right

-A flat drop with impact on the left

-A flat drop with impact on the bottom

-A flat drop with impact on the top

After test, there is no electrical and mechanical damage permitted.

| 1.2.4 Packing Configuration |  |  |  |
| --- | --- | --- | --- |
| 1.2.4.1 Container Specification |  |  |  |

| a. Shipping Container |  |  |  |  |
| --- | --- | --- | --- | --- |
| Container Type |  | 20'*8'*8'6" Steel | 40'*8'*8'6" Steel | 40'*8'*9'6" |
|  |  |  |  | High Cube Steel |
|  | Gross | 24,000 | 30,480 | 30,480 |
| Weight(Kgs) | Tare | 2,370 | 4,000 | 4,200 |
|  | Payload | 21,630 | 26,480 | 26,280 |
| Interior Measurement (mm) | Length | 5,898 | 12,031 | 12,031 |
|  | Width | 2,352 | 2,352 | 2,352 |
|  | Height | 2,394 | 2,394 | 2,699 |
| Volume(Cubic Meter) |  | 33.2 | 67.74 | 76.4 |
| Door opening(mm) | Width | 2,340 | 2,340 | 2,340 |
|  | Height | 2,280 | 2,280 | 2,585 |
| Useable Interior Dimension (Duducted pallet(110mm & Operating spce 50mm) | Length | 5,890 | 12,000 | 12,000 |
|  | Width | 2,330 | 2,330 | 2,330 |
|  | Height | 2,120 | 2,120 | 2,405 |

| b. Air Transport |  |  |  |  |
| --- | --- | --- | --- | --- |
| Container Type |  | Container (1) | Container (2) | Container (3) |
|  |  | 125"*96"*96" | 125"*96"*118" | 125"*88"*64" |
|  | Gross | 6,804 | 6,804 | 4,627 |
| Weight(Kgs) | Tare | 129 | 129 | 129 |
|  | Payload | 6,675 | 6,675 | 4,498 |
| Interior Measurement | Length | 3,048 | 3,048 | 3,048 |
| (mm) | Width | 2,260 | 2,260 | 2,082 |
|  | Height | 2,438 | 2,997 | 1,625 |
| Volume(Cubic Meter) |  | 17 | 19 | 11 |

| 1.2.4.2 Carton Specification |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Product | Net | 9.8 |  |  |  |
| Weight(Kgs) | Gross | 12.6 |  |  |  |
| Carton Interior | Length | 550 |  |  |  |
| Measurement(mm) | Width | 318 |  |  |  |
|  | Height | 589 |  |  |  |
| Carton External | Length | 570 |  |  |  |
| Measurement(mm) | Width | 338 |  |  |  |
|  | Height | 621 |  |  |  |

| 1.2.4.3 Pallet Specification |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| a. Dimension |  |  |  |  |  |  |
| Transport Type |  | Pallet A |  |  | Pallet B | Pallet C |
| Shipping Pallet | Length | 1140 |  |  | 1140 | X |
| Dimension(mm) | Width | 676 |  |  | 338 | X |
|  | Height | 110 |  |  | 110 | X |
| Air Transport Pallet | Length | 1140 |  |  | X | X |
| Dimension(mm) | Width | 676 |  |  | X | X |
|  | Height | 110 |  |  | X | X |

| 1.2.4.4 Container Carrying Capacity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| a, Shipping Container |  |  |  |  |  |  |
| Stowing Type |  | Quantity of products (sets) | Quantity of Products (sets) |  | Quantity of pallet (sets) |  |
|  |  | (Every container) | (Every Pallet) |  | (Every Container) |  |
|  | 20' | 204 | Pallet A: 12 |  | Pallet A: 16 |  |
|  |  |  | Pallet B: 6 |  | Pallet B: 2 |  |
| with pallet | 40' | 420 | Pallet A: 12 |  | Pallet A: 34 |  |
|  |  |  | Pallet B: 6 |  | Pallet B: 2 |  |
|  | 20' | 204 | X |  | X |  |
|  |  |  | X |  | X |  |
| without pallet | 40' | 420 | X |  | X |  |
|  |  |  | X |  | X |  |

b. Shipping Container Stowing Method

| c. Air Transport Container |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Container Type |  | Quantity of products (sets) (Every container) |  | Quantity of Products (sets) (Every Pallet) |  | Quantity of pallet (sets) (Every Container) |  |
| Container( 3 ) 3048 * 2082 * 1625 |  | 56 |  | Pallet A: 8 |  | Pallet A: 7 |  |
|  |  |  |  | Pallet B: X |  | Pallet B: X |  |
|  |  |  |  | Pallet C: X |  | Pallet C: X |  |

d. Air ransport Container Stowin Method

1.2.5 Electrostatic Discharge Requirements

The subject product must withstand 4 KV for contact discharge and 8 KV for air discharge of  Electrostatic Discharge and meet the acceptance criteria as specified in XXXISO  9241-3XXX--

should  be IEC 801-2 .

1.2.6 Safety Requirements

The display unit complies with the following safety standards and specifications.

-UL compliance....standard for information-processing and business equipment, UL 1950.

-CSA compliance...standard C22.2 No. 950-M89, data processing equipment.

-TUV compliance...EN60950 safety specification-business equipment.

-ZH1/618...German ergonomic standard.

-ISO9241-3 & ISO9241-8 ...Ergonomic Requirements of Visual Display.

-Demko...EN60950.

-Nemko...EN60950.

-Semko....EN60950.

-Fimko.N60950.

1.2.7 EMI Requirements

1. This display unit complies with the following EMC rules and regulations.

-FCC compliance...FCC Rule, Part 15, Subpart B, Class B.

-VCCI compliance...VCCI Rule, Class-2.

-CE Mark Compliance... 89/336/EEC.

EN50082-1,  IEC801-2, IEC801-3, IEC801-4.

EN50081-2, EN55022, Class B.

EN60555-2, EN60555-3.

-DNSF compliance...EN55022, Class B.

-MPR2 compliance

-TCO’99

2. The sample for EMI agency approval should be under 3 dB of the limit.

The production pilot run units should be under 3 dB of the limit.

The mass production units should be under 1 dB of the limit.

1.2.8 Acoustical Noise

With the display operating, the issue of sound measured is contained within 40dB/A in the

Audible field.

1.2.9 Reliability

1. The prediction MTBF of display unit shall be greater than 15,000 hours. ( at 25 degree C)

2. Lamp life time : 15,000 hours at following condition.

a.) Ambient temperature : 25±5 ℃

b.) Lamp current : 6mA or less

c.) The CFL life will be until the luminance 100 cd/m2 or more at maximum 
    white luminance.

1.2.10 Mechanical Design for TCO 99:

1) Front Frame Reflectance:

* diffuse reflectance: > 20%

* Gloss <= 30% gloss unit

2) Labeling of plastics:

Plastic weight > 25g shall be marked in accordance with ISO11469 & ISO 1043-4.

3) Variety of Plastic:

All plastic components that weight > 100g shall be made from the same type of plastic.

4) Painting of Plastic:

*Any plastic components that weight >100g shall not be painted lacquer or vanished, so that the paint,

lacquer or vanish in dry matter exceed 1 weight-% of the plastic component.

* Mould decoration (IMD) is not allowed

* All paints, lacquers, vanishes or colour additives used shall be declared by the type and mount.

5) Metallization of Plastic Housing:

* Metallization is not allowed.

6) Plastic components

Component > 25g shall not contain retardants of organically bound chloride or bromide.

1.2.11 Environment Protection Design :

Product is Per ES 715-c49 Environment Design Guide

1.3. Input / Output Signal Specification

1.3.1 Input Signal Requirements

1.3.1.1 Signal cable (Directly attached to unit)

1.3.1.1.1  Video Inputs:

15pin D-sub connector is on the captive signal cable for IBM VGA, 8514A or compatible

graphic adapters. The pin assignment of  this connector is described as below:

* 15pin D-sub female

1  RED VIDEO

2  GREEN VIDEO

3  BLUE VIDEO

4  GROUND

5  GROUND

6  RED GROUND

7  GREEN GROUND

8  BLUE GROUND

9  PC5V

10  SYNC GROUND

11  GROUND

12  SDA

13  H SYNC (H+V)

14  V SYNC

15  SCL

1.3.1.1.2 Cable length:

1500mm +/- 40mm

1.3.1.2 Video signals:

RGB separate, Analog 0.7Vp-p/75 Ohm

1.3.1.3 Sync signals:

- H/V separate, TTL level

- H/V composite, TTL level

1.3.2 Function

1.3.2.1 Multi-scan

This I/F board is designed to operate in any of the following video mode.

| Input display data |  |  |  |  | Multi-scan operation |
| --- | --- | --- | --- | --- | --- |
| Resolution | Horizontal Frequency (KHz) | Vertical Frequency (Hz) | Dot Clock Frequency (MHz) | Remark | Actual display resolution |
| ◆640x350 | 31.47(P) | 70.08(N) | 25.17 | DOS | full screen 1280x1024 |
| ◆640x350 | 37.86(P) | 85.10(N) | 31.5 | VESA |  |
| 720x400 | 31.47(N) | 70.08(P) | 28.32 | DOS |  |
| ◆720x400 | 37.93(N) | 85.00(P) | 35.5 | VESA |  |
| 640x480 | 31.47(N) | 60.00(N) | 25.18 | DOS |  |
| 640x480 | 35.00(N) | 67.00(N) | 30.24 | Macintosh |  |
| ◆640X480 | 37.86(N) | 72.80(N) | 31.5 | VESA |  |
| 640X480 | 37.50(N) | 75.00(N) | 31.5 | VESA |  |
| 640X480 | 43.27(N) | 85.00(N) | 36.00 | VESA |  |
| ◆800x600 | 35.16(P) | 56.25(P) | 36.00 | VESA |  |
| ◆800x600 | 37.88(P) | 60.32(P) | 40.00 | VESA |  |
| ◆800x600 | 48.08(P) | 72.19(P) | 50.00 | VESA |  |
| 800X600 | 46.87(P) | 75.00(P) | 49.5 | VESA |  |
| 800X600 | 53.67(P) | 85.10(P) | 56.25 | VESA |  |
| 832x624 | 49.72(N) | 74.55(N) | 57.29 | Macintosh |  |
| 1024x768 | 48.36(N) | 60.00(N) | 65.00 | VESA |  |
| 1024x768 | 56.48(N) | 70.10(N) | 75.00 | VESA |  |
| 1024x768 | 60.02(P) | 75.00(P) | 78.75 | VESA |  |
| ◆1024X768 | 35.52(P) | 86.95(P) | 44.96 | Interlace |  |
| 1024X768 | 60.24(N) | 74.93(N) | 80.00 | Macintosh |  |
| 1280X1024 | 64.00(P) | 60.00(P) | 108.00 | VESA |  |
| 1280X1024 | 80.00(P) | 75.00(P) | 135.00 | VESA |  |

Notes  (1) : The timing with “¨”symbol will not be adjusted in the factory.

(2) : If the incoming display mode is not supported by this I/F board listed above, the up side

of OSD window will show “Non Preset Mode” as below to indicate it. The picture won’t

probably be good and clear.

(3) OSD will show “No Signal” picture as below to indicate and color of LED will turn to

amber if no display mode inputs.

No Signal

(4) Some signals from graphics board may not function properly.

(5) “P”, “N” stands for “Positive”, “Negative” polarity of  incoming timing.

1.3.3 Number of display colors : 16M color number

1.3.4Adjustment function

1.)Luminance: Contrast and Brightness adjustment.

2.) Geometry: Horizontal Position, Vertical Position, Phase and Pixel Clock adjustment.

3.) Color Adjustment: Recall preset R,G,B color, Red, Green and Blue adjustment.

4.) Input signal information: Recall preset geometric values.

5.) Miscellaneous: OSD Position adjustment, Zoom(on/off) and Language Selection.

1.3.5 Plug & Play DDC1/2B Support

When the DDC function is supported, it is necessary to input all EDID data(128 bytes) by using

SCL and SDA signal of connector CNCR. These signals are directly connected to DDC IC

(24C21 by Atmel) in this I/F  board. It is recommended to check the specification of 24C21.

DDC Table

| Address | Data |  | Description |  |  |
| --- | --- | --- | --- | --- | --- |
| 00 | 00 |  |  |  |  |
| 01 | FF |  |  |  |  |
| 02 | FF |  |  |  |  |
| 03 | FF |  | Header |  |  |
| 04 | FF |  |  |  |  |
| 05 | FF |  |  |  |  |
| 06 | FF |  |  |  |  |
| 07 | 00 |  |  |  |  |
| 08 | 06 |  | ID Manufacturer Name = API |  |  |
| 09 | 09 |  |  |  |  |
| 0A | 11 |  | ID Product Code = FP855 |  |  |
| 0B | 76 |  | (Vender Assigned code) |  |  |
| 0C | * |  | ID Serial Number |  |  |
| 0D | * |  | 32 bits serial no. |  |  |
| 0E | * |  | (use 0 if n/a) |  |  |
| 0F | * |  |  |  |  |
| 10 | * |  | Week of Manufacture (0-53),use 0 if n/a |  |  |
| 11 | * |  | Year of Manufacture (year - 1990) |  |  |
| 12 | 01 |  | EDID version |  |  |
| 13 | 02 |  | Revision |  |  |
| 14 | 0C |  | Video Input Define |  |  |
| 15 | 24 |  | Max. H. Image Size (cm) |  |  |
| 16 | 1D |  | Max. V. Image Size (cm) |  |  |
| 17 | AA |  | (gamma*100) - 100 |  |  |
| 18 | E8 |  | DPMS |  |  |
| 19 | 16 |  | Red Green Bits Rx1Rx0Rxy1Ry0Gx1Gx0Gy1Gy0 |  |  |
| 1A | 06 |  | Blue White Bits Bx1Bx0By1By0Wx1Wx0Wy1Wy0 |  |  |
| 1B | A2 |  | Red x bit9-2 |  |  |
| 1C | 58 |  | Red y bit9-2 |  |  |
| 1D | 49 |  | Green x bit9-2 |  |  |
| 1E | 9D |  | Green y bits9-2 |  |  |
| 1F | 24 |  | Blue x bit9-2 |  |  |
| 20 | 17 |  | Blue y bit9-2 |  |  |
| 21 | 4F |  | White x bit9-2 |  |  |
| 22 | 58 |  | White y bit9-2 |  |  |
| Address | Data |  | Description |  |  |
| 23 | BF |  | Established Timing I |  |  |
| 24 | FF |  | Established Timing II |  |  |
| 25 | 00 |  | Established Timing III |  |  |
| 26 | 81 |  | Standard Timing Identification |  |  |
| 27 | 80 |  | #1 1280X1024 60HZ |  |  |
| 28 | 45 |  | #2 |  |  |
| 29 | 59 |  | 800X600 85Hz |  |  |
| 2A | 31 |  | #3 |  |  |
| 2B | 59 |  | 640X480 85HZ |  |  |
| 2C | 01 |  | #4 |  |  |
| 2D | 01 |  |  |  |  |
| 2E | 01 |  | #5 |  |  |
| 2F | 01 |  |  |  |  |
| 30 | 01 |  | #6 |  |  |
| 31 | 01 |  |  |  |  |
| 32 | 01 |  | #7 |  |  |
| 33 | 01 |  |  |  |  |
| 34 | 01 |  | #8 |  |  |
| 35 | 01 |  |  |  |  |
| 36 | D5 |  | Detailed Timing Description # 1 |  |  |
| 37 | 09 |  |  |  |  |
| 38 | 80 |  | (640X350, 70Hz) |  |  |
| 39 | A0 |  | Pixel clock : 25.175MHz |  |  |
| 3A | 20 |  |  |  |  |
| 3B | 5E |  |  |  |  |
| 3C | 63 |  |  |  |  |
| 3D | 10 |  |  |  |  |
| 3E | 10 |  |  |  |  |
| 3F | 60 |  |  |  |  |
| 40 | 52 |  |  |  |  |
| 41 | 08 |  |  |  |  |
| 42 | 04 |  |  |  |  |
| 43 | 03 |  |  |  |  |
| 44 | 00 |  |  |  |  |
| 45 | 08 |  |  |  |  |
| 46 | 06 |  |  |  |  |
| 47 | 1C |  |  |  |  |
| Address | Data |  | Description |  |  |
| 48 | 4E |  | Detailed Timing Description # 2 |  |  |
| 49 | 0C |  |  |  |  |
| 4A | 80 |  | 640X350 |  |  |
| 4B | C0 |  | 37.86kHz/85Hz |  |  |
| 4C | 20 |  |  |  |  |
| 4D | 5E |  |  |  |  |
| 4E | 5F |  |  |  |  |
| 4F | 10 |  |  |  |  |
| 50 | 20 |  |  |  |  |
| 51 | 40 |  |  |  |  |
| 52 | 03 |  |  |  |  |
| 53 | 08 |  |  |  |  |
| 54 | 0E |  |  |  |  |
| 55 | CB |  |  |  |  |
| 56 | 10 |  |  |  |  |
| 57 | 00 |  |  |  |  |
| 58 | 00 |  |  |  |  |
| 59 | 1A |  |  |  |  |
| 5A | DE |  | Detailed Timing Description # 3 |  |  |
| 5B | 0D |  |  |  |  |
| 5C | D0 |  | 720X400 |  |  |
| 5D | D8 |  | 37.93KHz/85Hz |  |  |
| 5E | 20 |  |  |  |  |
| 5F | 90 |  |  |  |  |
| 60 | 2E |  |  |  |  |
| 61 | 10 |  |  |  |  |
| 62 | 24 |  |  |  |  |
| 63 | 48 |  |  |  |  |
| 64 | 13 |  |  |  |  |
| 65 | 00 |  |  |  |  |
| 66 | 0E |  |  |  |  |
| 67 | CB |  |  |  |  |
| 68 | 10 |  |  |  |  |
| 69 | 00 |  |  |  |  |
| 6A | 00 |  |  |  |  |
| 6B | 1C |  |  |  |  |
| Address |  | Data |  | Description |  |
| 6C |  | 00 |  | Detailed Timing Description # 4 |  |
| 6D |  | 00 |  |  |  |
| 6E |  | 00 |  | This Detailed Timing Description |  |
| 6F |  | FE |  | ASCII String “Monitor” |  |
| 70 |  | 00 |  |  |  |
| 71 |  | 4D |  |  |  |
| 72 |  | 6F |  |  |  |
| 73 |  | 6E |  |  |  |
| 74 |  | 69 |  |  |  |
| 75 |  | 74 |  |  |  |
| 76 |  | 6F |  |  |  |
| 77 |  | 72 |  |  |  |
| 78 |  | 0A |  |  |  |
| 79 |  | 20 |  |  |  |
| 7A |  | 20 |  |  |  |
| 7B |  | 20 |  |  |  |
| 7C |  | 20 |  |  |  |
| 7D |  | 20 |  |  |  |
| 7E |  | 00 |  | Extension Flag |  |
| 7F |  | * |  | Check sum |  |

1.3.6 Power Supply Requirements

1.3.6.1 Input Power Requirements

(1) Input Voltage Range
	 The unit shall meet all the operating requirements with an input voltage range of 90~264 Vac .

(2) Input Current
           Maximum
          Input Current		  Measuring Range
         (MAX) 2 Arms            90Vac ～ 264Vac

(3) Frequency Range

The unit shall operate within a frequency range of 47Hz to 63Hz.

(4) Inrush Current

Power supply inrush current shall be less than the ratings of its critical components(including

Power switch, fuse, rectifiers and surge limiting device) for all conditions of line voltage.

(5) Regulator Efficiency

70% minimum (measuring at 115Vac and full load)

(6) Power line Transient Immunity

The power supply shall function properly after being subjected to a 0.3us/1.2us, 2000 volts

high peak pulse, or 5ns/10ns,1500 volts fast peak pulse applied either differentially or single

endedly to line and neutral at any phase of the power line voltage and shall not cause an unsafe

or unrecoverable errors.

(7) Maximum power consumption ＜65 Watts (without USB hub).

Maximum power consumption < 75 Watts ( with USB hub )

Power saving mode ＜5 Watts

(8) Safety concern : 1. Earth leakage current < 1mA ( 1.5k ohm )

2. Electric strength > AC 1200V ( 1min )

3. Insulation resistance > 10Mohm ( 500V )

1.3.6.2 Output Power Requirements

The power circuit shall supply DC power outputs as followings:

Output	           Nominal	     Regulation      Load Current Range
       1                12V (Inv.)           ±3%	           0 ~ 3.3A

2                12V                ±3%	           0 ~ 900mA

3                 5V-F              ±3%               0 ~ 200mA

4                 5V                ±3%	           0 ~ 500mA

5               3.3 V                ±3%               0 ~ 800mA
       6                 5V ( USB)          ±3%               0 ~ 2A

1.3.6.3 AC Power Inlet

The display unit shall be supplied with an AC power NICOON NC-174(or equivalent), to be located

at the rear of the LCD Monitor.

1.3.6.4 Input Power Cord

Each AC adapter unit shall be supplied with a input power cord, with length of 1800mm king cord

KC-003 or equivalent.

1.3.6.5 Power Management

| Mode | H/Vsync | Power consumption | LED Color (Status) | Recovery Time |
| --- | --- | --- | --- | --- |
| Normal | Both exist | < 65W (without USB hub) | Green (Normal) | -- |
| off | None or only one exist | < 5W (without USB hub) | Amber | 3 sec |

1.3.7 LCD panel optical Characteristics

| ITEM | SYMBOL | CONDITION | MIN. | TYP. | MAX. | UNIT | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Viewing Angle Range | θR | CR>=10, =0° | 60 | 80 | - | Degree | See Note1 |
|  | H | CR>=10, θ=0° | 60 | 80 | - | Degree | See Note1 |
|  | θL | CR>=10, =0° | 60 | 80 | - | Degree | See Note1 |
|  | L | CR>=10, θ=0° | 60 | 80 | - | Degree | See Note1 |
| Contrast Ratio | CR |  |  | 300 | - |  | See Note2 |
| Response time | Trising | 10% - 90 % | - | 30 |  | ms |  |
|  | Tfalling | 90% - 10% | - | 30 |  | ms |  |
| Chromaticity of white | Wx |  |  | 0.310 |  |  | +- 0.030 |
|  | Wy |  |  | 0.346 |  |  | +- 0.030 |
| Chromaticity of red | Rx |  |  | 0.633 |  |  | +- 0.040 |
|  | Ry |  |  | 0.345 |  |  | +- 0.030 |
| Chromaticity of green | Gx |  |  | 0.287 |  |  | +- 0.030 |
|  | Gy |  |  | 0.616 |  |  | +- 0.030 |
| Chromaticity of blue | Bx |  |  | 0.141 |  |  | +- 0.030 |
|  | By |  |  | 0.090 |  |  | +- 0.040 |
| Luminance of white | YL |  |  | 235 |  | cd/㎡ |  |

Note 1 : Definition of View Angle Range

Note 2 : Definition of Contrast Ratio(CR)

CR = (luminance with all pixels white) / (luminance with all pixels black)

1.4. Functional specification

All the tests to verify specifications in this section must be performed under the following standard conditions unless otherwise noted. The standard conditions are:

-Temperature : 25 +-5 degree Celsius
-AC line input voltage : 90Vac to 264Vac, 50Hz or 60Hz

-Warm-up time : 30 minutes minimum

1.4.1 Display Quality

1.4.1.1 Display Data Area (with full white pattern)

(1) Horizontal	359.0 mm

(2) Vertical  	287.2 mm

1.4.1.2 Video Performance

(1) Resolution :  1280 X 1024  pixels Maximum

(2) Contrast ratio : 300(Typ.)

(3) Response time : 50ms(Typ.)

(4)Viewing angle : U/D:80°  R/L:80° (Contrast ratio >= 10)

(5)CIE Coordinate: White (0.310, 0.346)        Tolerance : +/-  0.030

(6) Display color:  16 Millions  color number

1.4.1.3 Light Output

Brightness rating : 235cd/㎡  Typ.

1.4.1.4 Brightness Adjustment Range

At contrast ratio control set at maximum level, adjusting Brightness control from minimum to

maximum position, the light output of WHITE pattern shall be increased more than 40cd/㎡.

1.4.2 Audio Quality

1.4.2.1 Preamp + Poweramp:

(1) Output Power :	0.5 W rms/CH @ 1KHz, with 0.735Vrms Sin wave input.

(2) THD (@ 0.5W) : 	<1%

(3) Input Impedance :	50KΩ

(4) S/N ratio : 	50db

1.4.2.2 Speaker Driver:

(1) Nominal Impedance:		8 Ohm

(2) Maximum Input Power:		1 W/CH

(3) Frequency Response:		12KHz

(4) Mean Sound Pressure Level:	83 +/- 3dB

(5) Size:		28x40mm

(6) Magnetic Shield:		Required

1.4.2.3 Audio Controls:

(1)Volume

1.4.2.4 Microphone:

(1) Sensitivity:		-68 +/- 3dB (f=1KHz, 0 dB=1V/μbar)

(2) Directivity:	  	Omni-directional

(3) Frequency Response: 	100Hz ~ 16KHz

1.5. Physical Specifications

1.5.1 Physical Dimension & Appearance

1.5.1.1 Overall Dimensions:    457mm (W) X 468.5mm (H) X 217.4mm (D)

1.5.1.2 Outer Appearance: see  Fig.1

1.5.2 Construction and Materials on outer surface

(1) Materials: Plastic

(2) Color: White

To be defined for OEM

1.5.3 Base

(1) Tilt: 0/+25 degrees

1.5.4 Marking & Labels

1.5.4.1 Reference Label (Rear panel)

(1) Reference numbers

(2) Manufacture data

(3) Agency Approvals

(4) Power Ratings

1.5.4.2 Controls & Connectors

(1) AC power cord input: abbreviated labels

(2) User's Controls: standard print

1.5.5 Packaging

1.5.5.1 Carton Dimension:570mm (L) X338mm (W) X 621mm (H) ( LCD monitor )

1.5.5.2 Shipping Weight: 12.6kg ( LCD monitor )

1.5.5.3 Shipping Container LCD Monitor1:	420 sets per 40 feet container with pallet.

420 sets per 40 feet container without pallet.

204 sets per 20 feet container with pallet.

204 sets per 20 feet container without pallet.

1.6. Maintainability Specifications

1.6.1 General & Requirements

1.6.1.1 Installation: From outside of unit with standard tools and documentation provided to user.

1.6.1.2 Periodic Maintenance: No periodic maintenance is required.

1.6.1.3Repair & Calibration: Require spare modules or components as specified as followings:

(1) I/F Board  ASSY

(2) SPS Board  ASSY

(3) Control Board  ASSY

(4) Inverter board

(5) USB board(optional)

1.6.2 Mean Time to Repair

1.6.2.1 Module Level: Less than 10 minutes

1.6.2.2 Component Level: Less than 15 minutes

1.6.3 Accessibility

1.6.3.1 General:

All panels, covers, and major assemblies are removable without disruption of permanent mounting

or fasteners.

1.6.3.2 Outside Cabinet, access to the following elements

-Operating Controls

-DC Jack Connector

1.6.3.3 Cover Removal, Access

All sub assemblies and internally adjustable components may be accessed by removing the base  and
the rear cover .

1.6.4 Equipment & Tools Required

1.6.4.1 Standard Test Equipment

(1) Voltmeter

(2) Dual trace oscilloscope

(3) Hand tools as required

(4) Computer with IBM VGA , 8514A or compatible graphic adapter

1.6.4.2 Documentation

A service manual will be available which covers all service requirements. A users manual

written in  Japanese German, Italian, Spanish, France and English will be available to ship with

the product.

Chapter 2

CIRCUIT OPERATION THEORY

2.1 Introduction:

The FP855 is an 18.1" SXGA(1280x1024) 24 bits color TFT LCD monitor with multi-media

function and an optional USB hub. It’s an analog interface LCD monitor with an undetachable

15 pins D-sub signal cable and it’s compliant with VESA specification to offer a smart power management and power saving function. It also offers OSD menu for users to control the adjustable

items and get some information about this monitor, and the best function is to offer users an easy

method to set all adjustable items well just by pressing one key, we called it “i-key”(intelligent key)

which can auto adjusting all controlled items.

2.2 Block diagram

The FP855 consists of a head and a stand(base).  The head consists of a LCD module with 6 lamps, an Inverter, an Interface bard, a DC-DC board, a Control board and a microphone. The stand consists of an AC-DC board, an Audio board, two speakers and an optional USB hub. The block diagram is shown as below,

2.3 Circuit operation theory:

(A.) HEAD:

A-1.)Interface board diagram:

(a) Circuit operation theory:

A basic operation theory for this interface board is to convert analog signals of Red, Green

and Blue to digital signals of Red, Green and Blue, and by the internal PLL circuit of A/D

converter to generate the pixel clock to output to the scaling IC, then the scaling IC use the

SDRAM to be the frame buffer to process the different input signals which are operating in

the specification of the A/D converter and the scaling IC, finally the scaling IC output the

digital RGB data, the fixed frequency of Hsync, Vsync and pixel clock to LCD panel driver

IC by LVDS transmission.

(b) IC introduction:

1.) DDC(Display Data Channel) IC: We use ATMEL 24C21 to support DDC1/2B function,

it will store the information related to LCD monitor and PC can read them by “Vsync and

SDA” serial communication for DDC1 or I2C communication for DDC2B.

2.) Sync Processor: Because the A/D converter and the scaling IC of this board can’t separate

composite Sync into Hsync and Vsync by themselves, we use Mitsubishi M52347FP to

output the pure Hsync and Vsync respectively to them to solve this application problem.

3.)A/D converter: We use AD9884 of ADI which can support maximum 140MHz pixel rate to

convert analog signals of RGB to 24bits digital RGB signals and generate a pixel clock signal

by Hsync and its internal PLL circuit, then transmit these digital signals to Scaling IC.

4.)Scaling IC: We use MX88L282 of Macronix and by its frame buffer function to process the

digital RGB signals to maximum 75Hz refresh rate at 1280 by 1024 resolution(about 135MHz

pixel rate), it also support a ratio expansion function to display a full screen when the resolution

of input signals are less than 1280 by 1024.

5.)SDRAM: We use 3 pieces of 2M bytes SDRAM to be the frame buffer of the scaling IC to

process maximum 1280 by 1024 resolution of 24 bits RGB data.

6.)MCU: We use 80C32 with an external 27C256 EPROM to be the micro control unit, it controls

all the functions of this interface board, just like the mode detecting, OSD display setting, the

adjustable items, adjusted data storage, DPMS control, the external RS232 communication and

to control A/D converter and the scaling IC..

7.)EEPROM: We use 24C04 to store all the adjustable data except zoom on/off control, and we

divide it into two parts for the storage of factory settings and user settings.

8.)OSD IC: We use LSC3852 of Motorolla to do “On Screen Display” function.

9.)LVDS transmitter IC: We use two pieces of  75LVDS83 of TI to transmit all the digital data

to the LCD panel driver IC because this driver IC has two pieces of built-in 75LVDS84(LVDS

receiver IC).

A-2.) DC-DC board diagram:

- Fig. 1

(a)Buck Converter:

Fig. 2 shows the chopper regulator IC701(PQ1CF1) is used to convert +12V to +5V.

The Buck converter has ON/OFF control function, oscillation circuit, overheat protection,

overcurrent protection inside. Output +5V is decided by R702 and R703. C704 is the

compensation capacitor and D701 is the flywheel diode.

Fig. 2

(b)ON/OFF Control:

Fig. 3 Shows the On/Off control circuit. Q701 is a N channel MOSFET. Q702 and Q703

are NPN bipolar transistors. When MC set P/ON high level(+5V), Q702 turns on, Q703

turns off, Q701 turns on and output voltage (+5V, +3.3V) exist. Another situation (Power

Saving), MC set P/ON low level(0V). Q702 turns off, Q703 turns on, Q701 turns off and

output  voltage don’t exist.

Fig. 3

A-3.) Control board introduction:

The main parts of the control board are a push button, an encoder and a led.

(a)Push button: It’s a simple switch function,  pressing it for “ON” to do the auto

adjustment function, releasing it for “OFF” to do nothing.

(b)Encoder: It supports two functions, one is for wheel adjustment which can be controlled

in clockwise direction and counterclockwise direction, and the other is for push button

function. When we rotate the wheel, it will output the specific pulse to help us to know

its operating direction.

(c)Led: It indicates the DPMS status of this LCD monitor,

green light means DPMS on(Normal operating condition).

amber light means DPMS off(Power off condition).

A.) STAND(Base):

B-1.) AC-DC Board diagram:

Fig. 4

Fig 4.

(a)EMI Filter:

This circuit (fig. 5) is designed to inhibit electric and magnetic interference for meet FCC,

VDE, VCCI standard requirements

Fig. 5

Fig 5.

b)Rectifier and filter:

AC Voltage (90-264V) is rectified and filtered by BD601, C605 (See Fig6) and the DC

Output voltage is 1.4*(AC input). (See Fig.6)

Fig. 6

Fig 6.

(c )Switching element and Isolation power transformer:

When the Q601 turns on, energy is stored in the transformer. During Q601 OFF period,

the stored energy is delivered to the secondary of transformer. R604, C606 and D601 is a

snubber circuit. R605 is current sense resistor to control output power. (See Fig.7)

Fig. 7

Fig 7.

(d)Rectifier and filter:

D701 and C703 are  to  produce DC output.  L701  and C704  are   to  suppress  high

Frequency switching spikes. R717 is a dummy load. (See Fig. 8)

Fig. 8

Fig 8.

(e)Control circuit:

The current mode control IC UC3842B is used to control PWM. When the VCC terminal

of it gets 16V, IC601 turns on. +5V will be set up at Pin8 through soft start circuit includes

R620, C616, D608 and D609. Q602 controls IC601 to action or shutdown. When system goes

to power saving mode, Q602 turns off and IC601 shutdown. R618 and C615 generate a fixed

frequency sawtooth wave at Pin4. In the fact, the frequency has synchronized at 100MHZ

from TopSwitch IC605. (See Fig. 9)

Fig. 9

(f)Feedback circuit:

TLP721F is a optocoupler and TL431 is a shunt regulation. They are used to detect

the output voltage change and primany and secondary isolation. When output voltage changes,

the feedback voltage will be compared and duty cycle will be decided to control the correct

output voltage. (See Fig. 10)

Fig. 10

(g)TopSwitch(TOP224Y):

Fig. 11

Fig.11 shows the typical flyback application of TopSwitch. In the system, use the Top224Y

to be the switching element. It is more cost effective and provides several enhancements over

the other components. The drain terminal of it provides internal bias current during start-up via

an internal switched high-voltage current source. The control pin is error amplifier and feedback

current input pin for duty cycle control. There are two output voltages +12V and +5v in the

circuit. The +5V is used to supply the USB and another +12V is sent to DC-DC board to

convert several voltages.

B-2.)Audio Speaker(optional):

The Audio Speaker is consinst of a Audio board.  The Audio Speaker have  DC Volumn

control,Earphone jack , use 28mmX40mm Speaker (2W/per chennal),power supply from AC-DC

board and Audio input from PC  Audio  output (Line Out) .

(a)Power IC:Use Philips POWER IC TDA7053A.The IC are stereo BTL output amplifiers with DC

Volume control. The devices are designed for use in TV and monitor, but are also suitable for battery-Fed portable recorders and radios. Use +12V from AC-DC Board and connect speaker to offer 1W per chennal.

(b)DC Power Input:To supply +12V to be VCC source Voltage for TDA7053A and  connected

with AC-DC board.

(c)Audio Input:connect with PC Audio output in 3.5mm to 3.5mm signal line.

(d)Speaker:Use 8 ohm and 28mmX40mm speaker (1W/per chennal)

(e)DC Volumn Control:The two DC volume control stages are integrated into the input stages so

that no Coupling capacitors are required and a low offset voltage is still maintained. The minimum

supply voltage also remains low. If the DC volume control voltage falls below 0.4V, the device

will switch to the mute mode.

(H)Earphone jack:To connect 32 ohm earphone.

Chapter 3

Alignment procedure

1. Enter factory area (press “I-key” & “fly key” & “power on”)

2. Adjust RGB clamp level equal to 12

3. (1) do auto color adjustment it will show “pass”

(2) if fail adjust R=G=B=50 then do auto color adjustment

4. adjustment contrast equal to 50

5. save these parameter

6. press I_key adjust display area

7. Clear user area

8. save factory parameter then turn off power

9. turn on power enter user area

10. recall R,G,B parameter then save

| Input display data |  |  |  |  | Multi-scan operation |
| --- | --- | --- | --- | --- | --- |
| Resolution | Horizontal Frequency (KHz) | Vertical Frequency (Hz) | Dot Clock Frequency (MHz) | Remark | Actual display resolution |
| ◆640x350 | 31.47(P) | 70.08(N) | 25.17 | DOS | full screen 1280x1024 |
| ◆640x350 | 37.86(P) | 85.10(N) | 31.5 | VESA |  |
| 720x400 | 31.47(N) | 70.08(P) | 28.32 | DOS |  |
| ◆720x400 | 37.93(N) | 85.00(P) | 35.5 | VESA |  |
| 640x480 | 31.47(N) | 60.00(N) | 25.18 | DOS |  |
| 640x480 | 35.00(N) | 67.00(N) | 30.24 | Macintosh |  |
| ◆640X480 | 37.86(N) | 72.80(N) | 31.5 | VESA |  |
| 640X480 | 37.50(N) | 75.00(N) | 31.5 | VESA |  |
| 640X480 | 43.27(N) | 85.00(N) | 36.00 | VESA |  |
| ◆800x600 | 35.16(P) | 56.25(P) | 36.00 | VESA |  |
| ◆800x600 | 37.88(P) | 60.32(P) | 40.00 | VESA |  |
| ◆800x600 | 48.08(P) | 72.19(P) | 50.00 | VESA |  |
| 800X600 | 46.87(P) | 75.00(P) | 49.5 | VESA |  |
| 800X600 | 53.67(P) | 85.10(P) | 56.25 | VESA |  |
| 832x624 | 49.72(N) | 74.55(N) | 57.29 | Macintosh |  |
| 1024x768 | 48.36(N) | 60.00(N) | 65.00 | VESA |  |
| 1024x768 | 56.48(N) | 70.10(N) | 75.00 | VESA |  |
| 1024x768 | 60.02(P) | 75.00(P) | 78.75 | VESA |  |
| ◆1024X768 | 35.52(P) | 86.95(P) | 44.96 | Interlace |  |
| 1024X768 | 60.24(N) | 74.93(N) | 80.00 | Macintosh |  |
| 1280X1024 | 64.00(P) | 60.00(P) | 108.00 | VESA |  |
| 1280X1024 | 80.00(P) | 75.00(P) | 135.00 | VESA |  |

Chapter 4

TROUBLE  SHOOTING  GUIDE

4.1 No Picture or Picture is unstable:

4.1.1 Interface Board:

4.1.2. Control Board:

4.1.3. Power Board:

4.1.4. Audio Board:

| LCD FP550 Spare Parts Price FOR IBM |  |  |  |  |  | ISSUE DATE: August 2, 1999 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P/N: | 99.L0872.007 |  |  |  |  |
| ITEM | PART NO. | DESCRIPTION | LOCATION | PRICE |  |
| 1 | 55.70A04.001 | SPS BD FP850 (MI) |  |  |  |
| 2 | 04.03842.044 | IC VR UC3842BN DIP 8P | IC601 |  |  |
| 3 | 06.2R060.151 | DIODE BRI 2FBP06 600V 2A | BD601 |  |  |
| 4 | 13.R4734.03E | RES MOF 0.47 J 1W AF15 | R605 |  |  |
| 5 | 26.12001.132 | FUSE 2A 250V ST20 T/LAG SEMKO | F601 |  |  |
| 6 | 26.17001.310 | FUSE 7A 125V PICO F UL/CSA | FR701 |  |  |
| 7 | 74.00431.A3B | IC V.R ZR431 SOT-23 | IC701,IC702 |  |  |
| 8 | 06.02645.020 | FET MOS 2SK2645-01MR NC TO220F |  |  |  |
| 9 | 06.16010.110 | DIODE SB16CTQ100 16A100V TO-220 |  |  |  |
| 10 | 04.00224.030 | IC S.R TOP224Y TO -220 |  |  |  |
| 11 | 55.70A07.001 | DC-DC BD FP850 (MI) |  |  |  |
| 12 | 04.PQ1CF.030 | IC V.R PQ1CF1 DIP 3P | IC704 |  |  |
| 13 | 04.3RF23.030 | IC V.R PQ3RF23 3.3V TO-220 | IC815 |  |  |
| 14 | 55.70A08.001 | AUDIO BD FP850 (MI) |  |  |  |
| 15 | 04.07053.070 | IC AUDIO AMP TDA7053A DIP 16P | IC501 |  |  |
| 16 | 55.70A09.001 | USB BD FP850 (MI) |  |  |  |
| 17 | 04.03302.030 | IC V.R XC62AP3302LH 3.3V TO-92 | IC902 |  |  |
| 18 | 55.L0802.001 | CONTROL BD FP855 (MI) |  |  |  |
| 19 | 55.L0803.001 | INTERFACE BD FP855 (MI) |  |  |  |
| 20 | 02.02404.000 | IC EEPROM 24C04 4K-BIT DIP 8P | IC812 |  |  |
| 21 | 71.03852.00B | IC OSD LSC3852DW SO-W 24P | IC813 |  |  |
| 22 | 71.09884.009 | IC GRAPH DIGIT AD9884 MQFP128P | IC802 |  |  |
| 23 | 71.52347.00N | IC SYNC PROC M52347FP SOP 20P | IC801 |  |  |
| 24 | 71.80C32.60C | IC UCTRL 80C32X2 60MHZ PLCC40P | IC811 |  |  |
| 25 | 71.88282.A0E | IC FPD CTRL MX88L282 256P 3.3V | IC807 |  |  |
| 26 | 72.24C21.001 | IC EEPROM AT24C21 1K SO-N 8P | IC803 |  |  |
| 27 | 72.45161.A09 | IC DRAM 45161AG5-A10-7JF TSOP | IC808, IC810, IC809 |  |  |
| 28 | 73.07400.0N1 | IC CMOS 74ACT00 SO-N 14P | IC817, IC826 |  |  |
| 29 | 73.07474.0N1 | IC CMOS 74ACT74 SO-N 14P | IC822 |  |  |
| 30 | 73.07583.0H9 | IC XMITTER 75LVDS83 PSOP 56P | IC820, IC821 |  |  |
| 31 | 73.74123.L01 | IC TTL 74LS123 SO-N 16P (TI) |  |  |  |
| 32 | 73.74373.032 | IC TTL 74LS373 SO-W 20P | IC814 |  |  |
| 33 | 74.01117.03M | IC V.R LD1117-3.3 TO-252 | IC825 |  |  |
| 34 | 74.06300.03C | IC RESET V6300L HI-RES SOT233 | IC824 |  |  |
| 35 | 82.30024.301 | XTAL 24MHZ 30PF 30PPM 49US | X802 |  |  |
| 36 | 82.31431.001 | XTAL 14.318MHZ 32P 30PPM 49US | X801 |  |  |
| 37 | 84.04431.037 | FET MOS SI4431DY-T1 PC SO-8 | IC819 |  |  |
| 38 | 72.27256.053 | IC EPROM 27C256 120NS32K*8PLCC | IC815 |  |  |
| 39 | 33.70A04.001 | BKT-BASE SECC 2.0T FP850 |  |  |  |
| 40 | 42.70A04.001 | NECK-T PC+ABS 94V0 002 FP850 |  |  |  |
| 41 | 42.70A10.001 | UC EYE PC+ABS 94V0 002 850 |  |  |  |
| 42 | 50.L0804.001 | SIGNAL/C 15/16P W/C 2650MM 002 |  |  |  |
| 43 | 56.0770A.021 | LCDM 18.1" ITSX94/ IBM |  |  |  |
| 44 | 60.L0801.001 | ASSY SPEAKER 2W 8ohmJS |  |  |  |
| 45 | 60.L0802.001 | ASSY BZL PC+ABS 002 FP855/ACER |  |  |  |
| 46 | 42.00001.221 | BAG PE 900*900 ALL |  |  |  |
| 47 | 44.70A01.001 | CTN AB 570*338*621 FP855/ACER |  |  |  |
| 48 | 47.70A01.001 | CSN L EPS NATURE 850 |  |  |  |
| 49 | 47.70A02.001 | CSN R EPS NATURE 850 |  |  |  |
| 50 | 53.L0801.001 | MANUAL ACER FP855 TCO99 CD |  |  |  |
| * THIS PRICE LIST MAY BE SUBJECT TO CHANGE |  |  |  |  |  |  |  |
| WITH PRIOR NOTICE BY CSD OF API. |  |  |  |  |  |  |  |