KV32HS500, KV32HS510, KV32HV600, KV34HS510, KV34XBR800, KV36HS500, KV36HS510, KV36XBR800, KV40XBR800. Subject CRT socket P/N missing.

SYMPTOMS

The part number for J9001 the CRT socket on the CX board is missing from the service manual.

RESOLUTION

The part number is 1-451-544-11.

KV32HS500, KV32HV600, KV34XBR800, KV36HS500, KV36XBR800, KV40XBR800. Subject Bad Purity, not CRT.

SYMPTOMS

Picture had bad purity distortion. Replaced CRT same problem.

RESOLUTION

Found bad connections on the W Board. W Board replaced.

Best way to troubleshoot is to remove CN9100 & CN9102

SUPPORTING GRAPHICS

Parts

Part Number; Reference Number; Description; Symptom Code; Section Code; PCB Reference; Defect Code; Repair Code. A1400561A; 45; W Board; 1434; DFL; W; A; A.

KV32HS500, KV32HV600, KV34XBR800, KV36HS500, KV36XBR800, KV40XBR800. Subject Closed Caption not working.

SYMPTOMS

Closed Caption not working with local channels does work with the national channels. Other sets in home work, tried other CC modes same problem

CAUSE

Install circuit board mounting rail extenders that improve the A-board's reliability during handling of the TV. DESCRIPTION PART NUMBER RAIL EXTENDER KIT T-998-607-37 PVC PIPE GLUE PURCHASE LOCALLY* *Premier's Plastic Pipe Cement for PVC pipe and  low-pressure sprinklers is one example of this type of glue. The following references to left-side and right-side mean as the set is viewed from the front. Procedure: 1- Remove rear cabinet. 2- Remove A-board. 3- Identify the left-side and  right-side rail extenders (see illustration # 2).

RESOLUTION

- Does CC work for the sub picture? 
If so, there might be something wrong with the main tuner.
- How is the signal level for the local channels?
If the signal level is too low, the tuners will not be able to see the CC info.

The CCD  category in service mode does control CC decoding.
This data might have been corrupted. Check the data against the ones listed in the Service Data List, under CCD Category.

KV32HS500, KV32HV600, KV34XBR800, KV36HS500, KV36XBR800, KV40XBR800. Subject S Board has no schematic or listed in the Parts List..

SYMPTOMS

The block diagram show S Board but no breakdown schematic and also not in the parts list.

CAUSE

The correct P/N is X-403-763-13.

RESOLUTION

The schematic is correct, only the block is outdated. The block was a preliminary one we received from engineering. Component IC2501 on the S board were integrated into the M Board when the design was finalized.
The schematics are correct. See page  236 M Board 4/4
The Block will be updated.

KV32HS500, KV32HV600, KV34XBR800, KV36HS500, KV36XBR800, KV40XBR800. Subject No audio from the Audio Out (VAR/FIX).

SYMPTOMS

Customer is connecting a HiDef cable box to Video 6 component & audio left & right. The customer turns off internal speakers and fixed the output so he can have the audio go to the headphones. With this connect gets no audio from the Audio Out (VAR/FIX).  However when using the other composite inputs, there is sound that comes out fix var outs. Question is the Fix/Varible outs disable in using video 6 & 7.

RESOLUTION

The Audio Out (VAR/ FIX ) should works in using Video 5, 6 & 7 in all DA-4 chassis models.
In the case of the Customer's TV set , I think that the problem is related with the IC 2501 Audio processor circuit (TC94A04AF-014) on [M] board , this device  receives video 5 and video 6 audio inputs (Pin 46, 47(Left) ,50, 51 (Right)), process them and send the outputs through Pin 8, 10 to the IC2502 Audio Buff (IC2502).

KV32HS500, KV32HV600, KV36HS500. Subject Removal and installation of front panel control door..

SYMPTOMS

The method for removing and installing the front panel control door is not shown in the service manual.

RESOLUTION

To remove the door, refer to attached files named Removal step 1.jpg and Removal step 2.jpg.

To install the door, refer to attached files named Install step 1.jpg, Install steps 2 - 4.jpg, and Install step 5.jpg.

Removal step 1

Removal step 2

Install step 1

Install steps 2 – 4

Install step 5

KP57WV600, KP57WV700, KP65WV600, KP65WV700, KV32HS500, KV32HV600, KV34XBR800, KV36HS500, KV36XBR800, KV40XBR800. Subject TV won't display Memory Stick pictures, ''File Error'' appears on the screen..

SYMPTOMS

The TV won't display pictures stored on a Memory Stick. ''File Error'' appears when trying to view the pictures.

CAUSE

This will occur if the directory structure on the Memory Stick is not DCF compliant (DCF is an abbreviation for Design rules for Camera File systems). The error can also appear if the picture file names have been changed, or if the picture files have  been edited.

RESOLUTION

Refer the customer to the Memory Stick section of the owner's manual, or the Memory Stick supplement packaged with the TV, for instructions on the proper use of Memory Stick files and directories.

See the attached file ''Memory Stick Supplement  Sample'' for details related to the direct view models; rear projection models are similar.

NOTE: The new PRO memory stick will not work with current sets.

SUPPORTING DOCUMENTS AND GRAPHICS

KV32HS500, KV32HV600. Subject Incorrect warranty information packaged with TV..

SYMPTOMS

Early production sets included the warranty card for 2001 model year XBR models (1 year part and labor), but these models are covered under the standard warranty (1 year parts, 90 days labor).

RESOLUTION

The labor for KV-32HS500 sets up to and including serial number 8013628, and for KV-32HV700 sets up to and including 8002878, can be covered for a period up to 1 year from the date of original purchase.

KV32HS500, KV32HV600, KV34XBR800, KV36HS500, KV36XBR800, KV40XBR800. Subject 60 Hz bars through the picture.

SYMPTOMS

60 Hz bars through the picture. The bars disappear when the rear cover is removed.

RESOLUTION

Found the problem to be caused by the upper degaussing coil pinched to shield, from the pressure of the rear cabinet.
Remounted the coil to avoid being caught by the rear cabinet.

KP51WS500, KP57WS500, KP57WV600, KP57WV700, KP65WS500, KP65WV600, KP65WV700, KP65WV700, KV32HS500, KV32HV600, KV34XBR800, KV36HS500, KV36XBR800, KV40XBR800, SATHD200. Subject No picture when using DVI, not able to pick the best video format.

SYMPTOMS

Set might not display picture when hooked up to a STB with DVI output. Or if picture is displayed, STB will not be able to automatically pick the best video format to output to set.

As an example, for the SAT-HD200 (Sony's DirecTV/HD Receiver with  DVI functionality) the customer issue is that the box will not be able to operate in Auto-DVI mode.

CAUSE

Missing EDID data. 
The UD Board contains the timing preference for the signal received from the STB.

RESOLUTION

Replace the UD Board.

10/10/02 UD Boards in WRPC were placed on hold so they could have the software for the NVM updated. They should be available soon.

For this issue only, the warranty will be extended 2 years after the standard warranty period.

KP57WV600, KP57WV700, KP65WV600, KP65WV700, KV32HS500, KV32HV600, KV34XBR800, KV36HS500, KV36XBR800, KV40XBR800. Subject Memory slot P/N missing.

SYMPTOMS

Need the P/N for the memory slot which is not in the SM.

RESOLUTION

The memory slot is a complete assembly it's P/N is 1-816-402-11.
It is listed as Connector, Memory stick Ref CN7201.

The door is not a replaceable item as such.
Order the CN7201 connector.

On the Direct View models it's listed on the HM Board  Ref CN7201.

Part Number; Reference Number; Description; Symptom Code; Section Code; PCB Reference; Defect Code; Repair Code. 181640211; CN7201; Connector, Memory stick; 118X; INC; NONE; A; A.

KV32HS500, KV32HV600, KV34XBR800, KV36HS500, KV36XBR800, KV40XBR800. Subject Moving vertical bar while viewing 1080i.

SYMPTOMS

Vertical bar moving from right to left. The bar is about 4 to 5 inches wide and goes from right to left across the screen every second. This can be most easily seen in the standard or vivid picture mode. It's also more pronounced when brightness is  increased. Happens with either DVI or Component input.

CAUSE

B Board A1300-327-A Use in models: 
KV-32HV600, KV-34XBR800, KV-36XBR800, and KV-40XBR800.

B Board A-1300-327-A used in models:
KV-32HS500, KV-36HS500

RESOLUTION

The video path for 1080i will be changed.
Make sure the B Board has the components on the attached list. If not there replace the B Board.
Apply the data changes as per the attached list.

NOTE: 
1. Most sets in the market will already have these  parts mounted. Models produced between October through December 2002 might not have these parts. 

2. Units with the parts mounted just need to change the Service Mode settings. 

3. Boards in WRPC have these parts mounted. The part list  attached gives the list of components that must be on the board and should be used as a reference. If the parts are missing then the board should be replace. Adding the components to the B Board should not be attempted.

4. NVM (IC3089) of the  defective [B] board should be transfered to the new [B] board.

5. Check to see if IC-3414 is mounted under the shield cover component side of board. If there then you have the correct board with all components.

SUPPORTING DOCUMENTS AND GRAPHICS

KV32HS500, KV32HV600, KV34XBR800, KV36HS500, KV36XBR800, KV40XBR800. Subject M Board P/N correction.

SYMPTOMS

M Board P/N is not correct on page 295 of the S/M.

RESOLUTION

The correct P/N is A-1300-320-A

This was covered in S/M Correction-1

SUPPORTING DOCUMENTS AND GRAPHICS

KV32HS500, KV32HV600, KV34XBR800, KV36HS500, KV36XBR800, KV40XBR800. Subject Unit making crackling and popping noise.

SYMPTOMS

Customer complaint the unit make crackling and popping noise ! After 1 hour in the customer home I found out it just a squeezing noise from the TV cabinet when the temperature changed from hot to cold the plastic cabinet expands

RESOLUTION

Tech reported that he squeezes the two top corners of the Bezel and while squeezing moves each corner in the opposite direction at the same time. You will hear the cabinet make noise keep doing this procedure to the cabinet noise stops.

KV32HS500. Subject NS coil connection.

SYMPTOMS

Found cable coming from PC board mounted to the yoke disconnected. Where does this connector go? 
Where does the connector for the NS coil connect on the HS model ?

RESOLUTION

Should connect to CN9101 on W Board.

KV32HS500, KV36HS500. Subject What is the correct part number for the B Board.

SYMPTOMS

What is the correct part number for the B Board

RESOLUTION

SM part number for the B Board was revised.
The correct P/N for the B Board is:
A-1300-325-A
For models KV-32HS500, KV-36HS500 ONLY

| Model KV32HS500: Features |  |
| --- | --- |
|  | FD TRINITRON PICTURE TUBE |
|  | HI SCAN 1080I CHASSIS |
|  | DRC MULTI-FUNCTION(961I & 480P) |
|  | CINEMOTION REVERSE 3-2 PULLDOWN |
|  | FLEXIBLE TWIN-VIEW TWO TUNER PICTURE-AND-PICTURE |
|  | MID X (MULTI-IMAGE DRIVER X) CIRCUITRY |
|  | MEMORY STICK MEDIA STILL IMAGE PLAYBACK |
|  | "ARC" SHAPED DESIGN |
|  | CLEAR EDGE VM WIDEBAND VELOCITY MODULATION |
|  | 2 HD CAPABLE COMPONENT VIDEO INPUTS (Y/PB/PR) |
|  | 720P INOUT CAPABLE (1080I CONVERSION) |
|  | 3D DIGITAL COMB FILTER |
|  | AUTO 16:9 ENHANCED MODE |
|  | ID-1 DETECTION (480P) |
|  | TRUSURROUND SOUND (7.5W X 2) |
|  | 15W SUBWOOFER |
|  | FRONT: A/V INPUT WITH S-VIDEO |
|  | REAR: |
|  | COMPONENT VIDEO INPUT (2HD CAPABLE) |
|  | COMPOSITE VIDEO INPUT (3) |
|  | S-VIDEO INPUT (2) |
|  | AUDIO OUTPUT (FIXED/VARIABLE) |
|  | DUAL RF INPUTS |
|  | DVI--HDTV INTERFACE |

| Model KV32HS500: Specifications |  |
| --- | --- |
| DIMENSION | 35 3/8 X 27 1/8 X 24" |
| WEIGHT | 176 LBS 8 OZ |
| COMPONENT VIDEO | 2 REAR HD |
| COMPOSITE INPUT | 3 REAR/1 FRONT |
| SVIDEO INPUT | 2 REAR/1 FRONT |
| RF INPUT | 2 REAR |
| ENERGY STAR COM | YES |
| MONITOR OUT | 1 REAR |

KV32HV600, KV34XBR800, KV36XBR800, KV40XBR800. Subject Red tints when in HD mode.

SYMPTOMS

Red tints when in High Definition mode and regular TV pixelized

RESOLUTION

On HD up-close shots of people they look like everybody had bad sunburn. The reds were blooming. 

Replacing the B Board solved the problem.

SUPPORTING GRAPHICS

KV32HV600. Subject Ghosting on progressive scan, HD..

SYMPTOMS

Ghosting on progressive scan, HD.

RESOLUTION

This is do to the compress signal from Satellite or DVD disc. 

The affects of this high compression can be seen on all size TVs, however on 32 inch and above it becomes extremely noticeable, thus causing the most complaints. 

1. The distortion  is usually viewed as many small squares (tiling) instead of a clear picture, especially in areas that require a lot of detail (for example around the eyes). 
2. Ghosting or halo effect around people (usually seen around the shoulders or heads of  people). (DirecTV cause)
3. HD-100 was causing a red streak down a person's face. 
4. Solid walls or wood grain walls that should be stationary have movement. 
5. Also fast motion causes a similar artifact as number 1* (many small squares  (tiling) instead of a clear picture).

| Model KV32HV600: Features |  |
| --- | --- |
|  | FD TRINITRON PICTURE TUBE |
|  | HU-SCAN 1080I CHASSIS |
|  | DRC MULTIFUNCTION V1 (960I X 480P) |
|  | CINEMOTION REVERSE 3-2 PULLDOWN |
|  | FLEXIBLE TWIN-VIEW TWO TUNER PICTURE-AND-PICTURE |
|  | MID X (MULTI-IMAGE DRIVER X) CIRCUITRY |
|  | SCROLLING CHANNEL INDEX SYSTEM |
|  | MEMORY STICK MEDIA STILL IMAGE PLAYBACK |
|  | HD DETAILER WIDEBAND VIDEO AMP |
|  | CLEAREDGE VM WIDEBAND VELOCITY MODULATION |
|  | 2 HD CAPABLE COMPONENT VIDEO INPUTS (Y/PB/PR) |
|  | 720P INPUT CAPABLE(1080I CONVERSION) |
|  | 3D DIGITAL COMB FILTER |
|  | AUTO 16:9 ENHANCED MODE |
|  | ID-1 DETECTION (480P) |
|  | TRUSURROUND SOUND (7.5W X 2) |

| Model KV32HV600: Specifications |  |
| --- | --- |
| DIMENSION | 35 3/8 X 27 1/8 X 24" |
| WEIGHT | 176 LBS 8 OZ |
| COMPONENT VIDEO | 2 READ HD |
| COMPOSITE INPUT | 3 REAR/1 FRONT |
| SVIDEO INPUT | 2 REAR/1 FRONT |
| RF INPUT | 2 REAR |
| ENERGY STAR COM | YES |
| MONITOR OUT | 1 REAR |