Board	Inputs			Board Function			Outputs

U	TV Tuner		NTSC Input Switching		TV Output to rear panel

Composite Video	Component Video pass 		Composite S Video to BC4 bd.

Component Video	through				Component Video to BC bd.

A	VHF antenna		TV Tuner and control		Various

Interconnecting Video bds.

BC4	Composite Video (V)	Comb filter separates the Y&C	Separate Y&C Outputs for

or S Video (Y&C)	from the composite V (video)	the BC bd.

signal

BC	S Video (Y&C)		Creates a Main and Sub 	Main pic component video

Component video 	picture video path.  		Sub pic component video

inputs.			Channels video into

Main (DRC) & Sub pic

video paths.

BB	Main pic component	Switches pic location when	RGB and sync output to the

video.			NTSC video is input. Adds	LCD light box assembly.

Sub pic component	user menu into the video path.

video.

C	RGB and sync input	Independent RGB processing	Display outputs onto mirror.

into separate RGB LCD

panels.

NOTE: C board is part of the optical assembly.